# Thursday 3rd Dec 2020	(VD)

# First ethane sample

12:13	T = 278.2	P = 5.65 * 10-9 mbar

12:15	Cryo turned on

13:56	T = 20.9		P = 7.2 10*-10 mbar

14:15	MCT Detector cooled

14:31 Laser turned on

## Background scan(s) #1

15:20	BG20201203_01

512 scans	1 cm-1 res	signal 4.55
p = 7.1 x 10-10 mbar	T = 20.7 K

Ethane intro procédure :

- Make sure that gas line has been pumped and purge the whole time (up to the regulator – all valves open)

- Close black valve and then leaking valve at top

- To make sure that no impurity : flush the line once :

  - Open bottle and then regulator and close again

  - Open black valve and close

  - Make sure that gas cell chamber valve closed and close pump valve

  - Open leak valve and fill gas line with ethane

  - Open pump valve and let time for the system to get flushed (close leak valve)

- Open ethane bottle and set regulator pressure slightly above 1 bar

- Open gas cell chamber valve and close valve pump

- Open ethane black valve and slightly open the leak valve to fill the gas cell chamber

- When desired pressure reached in gas cell chamber, close gas cell chamber black valve

- Close leak valve and black valve in gas inlet set up

- Close bottle and regulator

- Open pump valve

- Open black valve and leak valve to pump the ethane line

      - Carry on with the XP  (If you haven’t blow up yet)

## Deposition #1: C2H6 @ 20.7 K

16:10 	20 min @ 1x10-7 mbar C2H6

- Initial Temperature: 20.7 K

- Initial pressure: 6.5*10-10 mbar

- Initial gas cell pressure: (before introducing ethane reading = 0.059) – 0.796

- Deposition pressure: 1x10-7

- Laser signal: 251.4.9 mV

- Deposition time: 20 min

- pressure after deposition 1.3 * 10-9 mbar

- final gas cell pressure = 0.765 Torr

16:31	laser Off / Head rotated

16:34	C2H6_2020_12_03_0001

512 scans	1 cm-1 res	signal 4.29
p = 8.5 x 10-10 mbar	T = 20.7 K

16:50	C2H6_2020_12_03_0002

512 scans	1 cm-1 res	signal 4.29
p = 8.5 x 10-10 mbar	T = 20.7 K

# 04/12

12:30	P = 5 * 10 -10 	T = 20.7

12:36	MCT Detector cooled

13:18	C2H6_2020_12_03_0003

512 scans	1 cm-1 res	signal 4.30
p = 5 x 10-10 mbar	T = 20.7 K

13:47	C2H6_2020_12_03_0004

512 scans	1 cm-1 res	signal 4.37
p = 5 x 10-10 mbar	T = 20.7 K

## Annealing to 30 K

All files: C2H6_2020_12_03__000# (see tables) 512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 14 :30 | _0005 | Warm-up | 4.35 |
| 14 :45 | _0006 | Iso1 | 4.35 |
| 15:00 | _0007 | Iso2 |  |

## Annealing to 40 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 15 :15 | _0008 | Warm-up |  |
| 15:30 | _0009 | Iso1 | 4.34 |
| 15:45 | _0010 | Iso2 | 4.33 |

## Annealing to 50 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 16 :00 | _0011 | Warm-up | 4.34 |
| 16:15 | _0012 | Iso1 | 4.32 |
| 16:30 | _0013 | Iso2 | 4.33 |

## Annealing to 60 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 16 :45 | _0014 | Warm-up |  |
| 17:00 | _0015 | Iso1 |  |
| 17:15 | _0016 | Iso2 | 4.32 |

## Annealing to 70 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 17 :30 | _0017 | Warm-up | 4.31 |
| 17:45 | _0018 | Iso1 |  |

18 :01	Heater turned off

18 :02 Cryo off

# 13/12 Shutting Down PAC

## 13:07	Turbo pump switched off

13:14	I close the leak valves on the gas lines (have been pumped and under vacuum)

13:16 	T = 278.5	P Chamber = 4.2 * 10 -8		P gas cell = 0.059 Torr

13:20 	P chamber = 1.8 * 10-7

13:23 	P Chamber = 1.7 * 10-6

13:26	P chamber = 9.6 *10-6

13:27 	P chamber= 2.4 * 10-5

## 13:25		N2 Filling procedure

13:25:	Close gas chamber valve

13:30	N2 valve open (1.1 bar)

13:30	Open gas black valve and let some n2 leak (to let the air on the line get pumped)

Needle valve open and P set at 5 * 10-2

{…}

P = 1.2 mbar

All valved close

## 14:35		Switch off electronics

Heater

Baratron

Main chamber gage

Spectro

# Friday 29th Jan 2020	(VD)

# First ethane sample

## Background scan(s) #1

11:27	BG20210129_01

512 scans	1 cm-1 res	signal 3.88
p = 1.3 x 10-9 mbar	T = 20.8 K

## Mixture preparation : C2H6 - ASW 1:10 ratio

XX:XX 	- Initial gas cell pressure: 0.055

XX:XX 	- H2O intro (0.5) = 0.485

XX:XX 	-C2H6 intro ( + 0.5) = 1.034

XX:XX 	-Gas cell pump (down to 0.1) = 0.106 when valve closed but equilibrate at 0.986

XX:XX 	- H2O intro (up to 1) = 0.953

Remarque:

## Deposition #2: C2H6 - ASW @ 20.8 K

12:12 	20 min @ 1x10-7 mbar C2H6 – ASW 1:10

- Initial Temperature: 20.8 K

- Initial pressure: 1.1*10-9 mbar

- Initial gas cell pressure: 0.953

- Deposition pressure: 1x10-7

- Laser signal: 251.9 mV

- Deposition time: 20 min

- pressure after deposition 1.2 * 10-8 mbar

- final gas cell pressure = 0.842 Torr

12:34	laser Off / Head rotated

12:38	C2H6_ASW_2021_01_29_0001

512 scans	1 cm-1 res	signal 3.83
p = 5 x 10-9 mbar	T = 20.8 K

12:55	C2H6_ASW_2021_01_29_0002

512 scans	1 cm-1 res	signal 3.83
p = 2.3 x 10-9 mbar	T = 20.8 K

## Annealing to 30 K

All files: C2H6_ASW_2021_01_29__000# (see tables) 512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 13 :30 | _0003 | Warm-up | 3.8 |
| 13 :45 | _0004 | Iso1 | 3.79 |
| 14:00 | _0005 | Iso2 | 3.79 |

## Annealing to 40 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 14:15 | _0006 | Warm-up | 3.78 |
| 14 :30 | _0007 | Iso1 | 3.77 |
| 14:45 | _0008 | Iso2 |  |

## Annealing to 50 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 15 :00 | _0009 | Warm-up | 3.77 |
| 15:15 | _0010 | Iso1 | 3.77 |
| 15:30 | _0011 | Iso2 |  |

## Annealing to 60 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 15 :45 | _0012 | Warm-up | 3.76 |
| 16 :00 | _0013 | Iso1 | 3.76 |

16 :15 Macro 20 scans (14 - 33)

# 30/01

11:30	P = 5 * 10 -10 	T = 60.1

11:42	MCT Detector cooled

12:00	C2H6_ASW_2021_01_29_0034

512 scans	1 cm-1 res	signal 3.66
p = 5 x 10-10 mbar	T = 60 K

12:45	C2H6_ASW_2021_01_29_0035

512 scans	1 cm-1 res	signal 3.84
p = 5 x 10-10 mbar	T = 60 K

## Annealing to 70 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 13 :00 | _0036 | Warm-up | 3.84 |
| 13:15 | _0037 | Iso1 | 3.83 |
| 13:30 | _0038 | Iso2 |  |

## Annealing to 80 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 13 :45 | _0039 | Warm-up |  |
| 14:00 | _0040 | Iso1 | 3.83 |
| 14:15 | _0041 | Iso2 | 3.83 |

## Annealing to 90 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 14 :30 | _0042 | Warm-up | 3.82 |
| 14:45 | _0043 | Iso1 | 3.81 |
| 15:00 | _0044 | Iso2 | 3.81 |

## Annealing to 100 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 15 :15 | _0045 | Warm-up | 3.81 |
| 15:30 | _0046 | Iso1 | 3.81 |
| 15:45 | _0047 | Iso2 | 0.10 |

## Annealing to 110 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 16:00 | _0048 | Warm-up | 3.79 |
| 16:15 | _0049 | Iso1 | 3.79 |
| 16:30 | _0050 | Iso2 |  |

## Annealing to 120 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 16:45 | _0051 | Warm-up | 3.79 |
| 17:00 | _0052 | Iso1 |  |
| 17:15 | _0053 | Iso2 | 3.78 |

## Annealing to 130 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 17:30 | _0054 | Warm-up | 3.79 |
| 17:45 | _0055 | Iso1 | 3.79 |
| 18:00 | _0056 | Iso2 |  |

## Annealing to 140 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 18:15 | _0057 | Warm-up | 3.78 |
| 18:30 | _0058 | Iso1 | 3.80 |
| 18:45 | _0059 | Iso2 | 3.79 |

## Annealing to 150 K

| time | File | Type | signal |
| --- | --- | --- | --- |
| 19:00 | _0060 | Warm-up | 3.78 |

# 19 : 15 Macro 30 scans

01/02 10 :30 cryo off / heater off

# Wednesday 24th Feb 2021	(VD) – water + ethane layer on top

13:58 	T = 278.6 K P = 9.7 * 10-10

13:58	Cryo-on

14:00	Laser On

15:33	T = 20.8 P = 5* 10-10 mbar

15:55 MCT detector cooled

## Background scan(s) #1

16:40	BG20210224_01

512 scans	res = 1 cm-1	signal = 3.71
               p = 5 * 10-10 mbar          T = 20.6K

17 :00 Head rotated (laser moved so put it back in place)

## Deposition #3.1: H2O @ 20K

11:40 	10 min @ 1x10-7 mbar H2O

- Initial Temperature: 20.6K

- Initial pressure: 5 * 10-10 mbar

- Initial gas cell pressure: 7.35 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  322.4 mV

- Deposition time: 10 min

- pressure after deposition 9.3 e-9

- final gas cell pressure = 6.56 Torr

17 :18 Head rotated, laser off

17:20	C2H6_ASW_2021_02_24_0001

512 scans	1 cm-1 res	signal 3.92
p = 4 x 10-9 mbar	T = 20.6 K

17 :35 Macro 40 scans

# 25/02

10:07	P = 5 * 10 -10 	T = 20.5

09:43	MCT Detector cooled – LASER on

10:15	C2H6_ASW_2021_02_24_0042

512 scans	1 cm-1 res	signal 3.90
p = 5 x 10-10 mbar	T = 20 K

10:45	BG20210224_02

512 scans	res = 1 cm-1	signal = 3.99
               p = 5 * 10-10 mbar          T = 20.5K

## Deposition #3.2: C2H6 @ 20K

12:28 	5 min @ 1x10-7 mbar C2H6

- Initial Temperature: 20.5K

- Initial pressure: 5 * 10-10 mbar

- Gas cell pressure empty: 0.055 (reading)

- Initial gas cell pressure: 7.86 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  324.8 mV

- Deposition time: 5 min

- pressure after deposition 1.3 * 10-9

- final gas cell pressure = 7.64Torr

12 :35 Head rotated, laser off

12:38	C2H6_ASW_2021_02_24_0043 (bg1)

512 scans	1 cm-1 res	signal 3.75
p = 5 x 10-10 mbar	T = 20.5 K

(VERY FEW ETHANE – NO PRELIMINARY PUMPING OF GAS LINE)

12:55	C2H6_ASW_2021_02_24_0044 (bg1)

512 scans	1 cm-1 res	signal 4.04
p = 5 x 10-10 mbar	T = 20 K

13:12	C2H6_ASW_2021_02_24_0045 (bg2)

512 scans	1 cm-1 res	signal 4.04
p = 5 x 10-10 mbar	T = 20.4 K

14:21	BG20210224_03

512 scans	res = 1 cm-1	signal = 4.02
               p = 5 * 10-10 mbar          T = 20.5K

14 :48 Laser On

## Deposition #3.3: C2H6 @ 20K

15:30 	5 min @ 1x10-7 mbar C2H6

- Initial Temperature: 20.5K

- Initial pressure: 5 * 10-10 mbar

- Gas cell pressure empty: 0.056 (reading)

- Initial gas cell pressure: 8.70 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  325.5 mV

- Deposition time: 5 min

- pressure after deposition 5 * 10-10

- final gas cell pressure = 8.62Torr

15 :37 Head rotated

15:40	C2H6_ASW_2021_02_24_0046 (bg1)

512 scans	1 cm-1 res	signal 4.04
p = 5 x 10-10 mbar	T = 20.5 K

16:00	C2H6_ASW_2021_02_24_0047 (bg3)

512 scans	1 cm-1 res	signal 4.04
p = 5 x 10-10 mbar	T = 20.4 K

16 :15 Macro 20 scan (bkg 1 )

# 26/02

9:44 P = 5 * 10-10 T = 20.3

9:49 MCT Detector cooled

10:45	C2H6_ASW_2021_02_24_0068 (bg1)

512 scans	1 cm-1 res	signal 
p = 5 x 10-10 mbar	T = 20.4 K

11:50	C2H6_ASW_2021_02_24_0069 (bg1)

512 scans	1 cm-1 res	signal 3.76
p = 5 x 10-10 mbar	T = 20.4 K

## Annealing to 30 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 14 :00 | _0070 | Warm-up | 3.68 |

+ Macro 7 scans

## Annealing to 40 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 16 :00 | _0078 | Warm-up | 3.66 |

16 :15  Macro 20 scans

# 27/02

9:47 P = 7.6 * 10-10 T = 40.0

10:00 MCT Detector cooled

10:30	C2H6_ASW_2021_02_24_0099 (bg1)

512 scans	1 cm-1 res	signal 3.71
p = 7 x 10-10 mbar	T = 40 K

10:45	C2H6_ASW_2021_02_24_0100 (bg1)

512 scans	1 cm-1 res	signal 3.71
p = 7 x 10-10 mbar	T = 40 K

## Annealing to 50 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 11 :00 | _0101 | Warm-up | 3.73 |

+ Macro 7 scans

## Annealing to 60 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 13 :00 | _0109 | Warm-up | 3.68 |

+ Macro 7 scans

## Annealing to 70 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 15 :00 | _0117 | Warm-up | 3.64 |

+ Macro 7 scans

## Annealing to 80 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 15 :00 | _0125 | Warm-up | 3.60 |

+ Macro 20 scans

# 28/02

14:22 P = 5 * 10-10 T = 80.0

14:30 MCT Detector cooled

14:45	C2H6_ASW_2021_02_24_0146 (bg1)

512 scans	1 cm-1 res	signal 3.49
p = 7 x 10-10 mbar	T = 80 K

15 :00 Macro 40 scans

# 01/03

09:47	P = 5 * 10 -10 	T = 80.0

09:53	MCT Detector cooled

10:25	C2H6_ASW_2021_02_24_0187 (bg1)

512 scans	1 cm-1 res	signal 3.64
p = 5 x 10-10 mbar	T = 80 K

10:45	C2H6_ASW_2021_02_24_0188 (bg1)

512 scans	1 cm-1 res	signal 3.68
p = 5 x 10-10 mbar	T = 80 K

## Annealing to 90 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 11 :00 | _0189 | Warm-up | 3.67 |

+ Macro 7 scans

## Annealing to 100 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 13 :00 | _0197 | Warm-up | 3.63 |

+ Macro 7 scans

## Annealing to 110 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 15 :00 | _0205 | Warm-up | 3.60 |

+ Macro 7 scans

## Annealing to 120 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 15 :00 | _0213 | Warm-up | 3.60 |

+ Macro 20 scans

# 02/03

09:50	P = 5 * 10 -10 	T = 120.0

10:02	MCT Detector cooled

10:30	C2H6_ASW_2021_02_24_0234 (bg1)

512 scans	1 cm-1 res	signal 3.51
p = 5 x 10-10 mbar	T = 120 K

10:45	C2H6_ASW_2021_02_24_0235 (bg1)

512 scans	1 cm-1 res	signal 3.67
p = 5 x 10-10 mbar	T = 80 K

## Annealing to 130 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 11 :00 | _0236 | Warm-up | 3.68 |

+ Macro 60 scans

12 :21 P = 8.7 * 10-9

12 :32	P = 9.3 *10-9

# 03/03

10:16	P = 3.8 * 10 -9 	T = 130.0

10:27	MCT Detector cooled

10:50	C2H6_ASW_2021_02_24_0297 (bg1)

512 scans	1 cm-1 res	signal 
p = 3.8 x 10-9 mbar	T = 130 K

11:12	C2H6_ASW_2021_02_24_0298 (bg1)

512 scans	1 cm-1 res	signal 
p = 3.8 x 10-9 mbar	T = 130 K

11:58	C2H6_ASW_2021_02_24_0299 (bg1)

512 scans	1 cm-1 res	signal 
p = 3.8 x 10-9 mbar	T = 130 K

12:40	C2H6_ASW_2021_02_24_0300 (bg1)

512 scans	1 cm-1 res	signal 3.66
p = 3.8 x 10-9 mbar	T = 130 K

## Annealing to 135 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 14 :00 | _0301 | Warm-up | 3.63 |

+ Macro 7 scans

## Annealing to 140 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 16 :00 | _0309 | Warm-up | 3.61 |

+ Macro 40 scans

# 04/03

11:13	P = 2.3 * 10 -8 	T = 140.0

11:20	MCT Detector cooled

12:25	C2H6_ASW_2021_02_24_0350 (bg1)

512 scans	1 cm-1 res	signal 3.64
p = 2.3 x 10-8 mbar	T = 140 K

## Annealing to 150 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 13 :00 | _0350 | Warm-up | 3.64 |

+ Macro 40 scans

05/03 14 :04 cryo stopped

# Wednesday 8th March 2021	(VD) – water + ethane Mixture 1:1

09:32 	T = 278.4 K P = 1.1 * 10-9

09:32	Cryo-on

09:32	Laser On

10:06	T = 180.6	P = 8 * 10-10

10:16 MCT detector cooled

10:30 Initial gas cell pressure 0.077  warm up using heat gun (2 min P rise 0.080)

10:54 Pgas cell = 0.076

## Mixture preparation : C2H6 - ASW 1:1 ratio

10:58 	- Initial gas cell pressure: 0.076

Procedure (LV = LEAK VALVE ; MV = MAIN VALVE)

- Water intro:

  - Close ethane line LV (P gas cell chamber decrease – leak ?)

  - P gas cell chamber stabilise at 0.055

  - Close water line (LV)

  - Close gas cell chamber

  - Open water MV

  - Close valve Pump

  - Open gas cell valve

  - Open water LV slowly up to desire value

XX:XX 	- H2O intro (0.5) = 0.516

  - Let the pressure equilibrate and close the gas cell chamber valve

  - Close water MV

  - Open water LV

  - Open pump valve and allow time for the system to pump

  - Open ethane LV (All open up to bottle)

  - Close regulator MV AND Lv IN THAT ORDER

  - Close pump valve

  - Open ethane bottle and regulator up to 1.1 bar, MV and LV (briefly LV and close)

  - Open pump valve (flush) and close

  - Open ethane LV and glas cell chamber valve (together and close gas cell chamber valve when ethane intro done (real quick))

XX:XX 	-C2H6 intro ( + 0.5) = 1.077

## Background scan(s) #1

11:35	BG20210308_01

512 scans	res = 1 cm-1	signal = 3.51
               p = 5 * 10-10 mbar          T = 20.6K

## Deposition #4: C2H6 - ASW @ 20.5 K

12:05 	20 min @ 1x10-7 mbar C2H6 – ASW 1:1

- Initial Temperature: 20.5 K

- Initial pressure: 5*10-10 mbar

- Initial gas cell pressure: 1.062

- Deposition pressure: 1x10-7

- Laser signal: 327.0 mV

- Deposition time: 20 min

- pressure after deposition 6 * 10-9 mbar

- final gas cell pressure = 1.008 Torr

12:30	laser Off / Head rotated

12:30	C2H6_ASW_2021_03_08_0001 (bg1)

512 scans	1 cm-1 res	signal 3.94
p = 3 x 10-9 mbar	T = 20.5 K

13:06	C2H6_ASW_2021_03_08_0002 (bg1)

512 scans	1 cm-1 res	signal 3.92
p = 9 x 10-10 mbar	T = 20.5 K

13:30	C2H6_ASW_2021_03_08_0003 (bg1)

512 scans	1 cm-1 res	signal 3.91
p = 7.5 x 10-10 mbar	T = 20.5 K

13:45	C2H6_ASW_2021_03_08_0004 (bg1)

512 scans	1 cm-1 res	signal 3.91
p = 7.5 x 10-10 mbar	T = 20.5 K

14 :00	 MACRO 40 scans

09/03/

P = 5 * 10-10 T = 20.4

09 :50	MCT Detctor cooled

10:00	C2H6_ASW_2021_03_08_0045

512 scans	1 cm-1 res	signal 3.76
p = 5 x 10-10 mbar	T = 20.4 K

10:30	C2H6_ASW_2021_03_08_0046

512 scans	1 cm-1 res	signal 4.00
p = 5 x 10-10 mbar	T = 20.4 K

11:00	C2H6_ASW_2021_03_08_0047

512 scans	1 cm-1 res	signal 
p = 5 x 10-10 mbar	T = 20.4 K

11:30	C2H6_ASW_2021_03_08_0048

512 scans	1 cm-1 res	signal 4.01
p = 5 x 10-10 mbar	T = 20.4 K

12 :00	Macro 10 scans

14:30	C2H6_ASW_2021_03_08_0059

512 scans	1 cm-1 res	signal 3.96
p = 5 x 10-10 mbar	T = 20.3 K

14:45	C2H6_ASW_2021_03_08_0060

512 scans	1 cm-1 res	signal 3.95
p = 5 x 10-10 mbar	T = 20.3 K

15 :00 MACRO 16 SCANS

18 :45 Macro 40 scans

10/03

10 :16 MCT Detector cooled T = 20.3 P = 5 * 10-10

10:30	C2H6_ASW_2021_03_08_0117 (bg1)

512 scans	1 cm-1 res	signal 
p = 5 x 10-10 mbar	T = 20.3 K

10:45	C2H6_ASW_2021_03_08_0118 (bg1)

512 scans	1 cm-1 res	signal 4.00
p = 5 x 10-10 mbar	T = 20.3 K

## Annealing to 30 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 11 :00 | _0119 | Warm-up | 4.03 |

+ Macro 7 scans

## Annealing to 40 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 13 :00 | _0127 | Warm-up | 4.03 |

+ Macro 7 scans

## Annealing to 50 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 15 :00 | _0136 | Warm-up |  |

+ Macro 7 scans

17 :00	Macro 4 scan

18 :00	Macro 4 scan

## Annealing to 60 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 19 :00 | _0151 | Warm-up |  |

+ Macro 7 scans

21 :02 MCT Detector cooled

21:30	C2H6_ASW_2021_03_08_0159 (bg1)

512 scans	1 cm-1 res	signal 3.83
p = 3.1 x 10-8 mbar	T = 60 K

21 :45 Macro 80 (exit at loop 60 up to scan )

11/03

11 :49 MCT Detector cooled

12:56

13:00	C2H6_ASW_2021_03_08_0220 (bg1)

512 scans	1 cm-1 res	signal 3.74
p = 2 x 10-8 mbar	T = 60 K

13:30	C2H6_ASW_2021_03_08_0221 (bg1)

512 scans	1 cm-1 res	signal 
p = 2 x 10-8 mbar	T = 60 K

## Annealing to 70 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 14 :00 | _0222 | Warm-up | 3.77 |

+ Macro 7 scans

## Annealing to 80 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 16 :00 | _0230 | Warm-up | 3.62 |

+ Macro 7 scans

## Annealing to 90 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 18 :00 | _0238 | Warm-up | 3.60 |

+ Macro 20 scans

12/03

09 :42	P = 5 * 10-10 T = 90K

09 :49 MCT Detector cooled

10:15	C2H6_ASW_2021_03_08_0259 (bg1)

512 scans	1 cm-1 res	signal 
p = 2 x 10-8 mbar	T = 90 K

10:30	C2H6_ASW_2021_03_08_0260 (bg1)

512 scans	1 cm-1 res	signal 3.70
p = 2 x 10-8 mbar	T = 90 K

## Annealing to 100 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 11 :00 | _0261 | Warm-up | 3.71 |

+ Macro 7 scans

## Annealing to 110 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 13 :00 | _0269 | Warm-up |  |

+ Macro 7 scans

## Annealing to 120 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 15 :00 | _0277 | Warm-up | 3.67 |

+ Macro 7 scans

## Annealing to 130 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 17 :00 | _0285 | Warm-up | 3.65 |

+ Macro 20 scans

13/03

12 :26	T = 130.0	P = 4.6 e-9

12 :35 MCT Detector cooled (and realigned)

13:00	C2H6_ASW_2021_03_08_0306 (bg1)

512 scans	1 cm-1 res	signal 3.70
p = 4.6 x 10-9 mbar	T = 130 K

15:45	C2H6_ASW_2021_03_08_0307 (bg1)

512 scans	1 cm-1 res	signal 3.30
p = 4.5 x 10-9 mbar	T = 130 K

15/03/2021

13 :10 P = 4 * 10-9	T = 130K

13 :20 MCT Detector cooled

13:45	C2H6_ASW_2021_03_08_0308 (bg1)

512 scans	1 cm-1 res	signal 
p = 3.6 x 10-9 mbar	T = 130 K

14:00	C2H6_ASW_2021_03_08_0309 (bg1)

512 scans	1 cm-1 res	signal 
p = 3.6 x 10-9 mbar	T = 130 K

14:30	C2H6_ASW_2021_03_08_0310 (bg1)

512 scans	1 cm-1 res	signal 3.58
p = 3.3 x 10-9 mbar	T = 130 K

14:45	C2H6_ASW_2021_03_08_0311 (bg1)

512 scans	1 cm-1 res	signal 3.58
p = 3.3 x 10-9 mbar	T = 130 K

Macro 4 scans

(15 :15 air pressure set up to 75 psi)

## Annealing to 140 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 16 :00 | _0316 | Warm-up | 3.58 |

+ Macro 7 scans

## Annealing to 150 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 18 :00 | _0324 | Warm-up | 3.59 |

+ Macro 20 scans

# Wednesday 24th March 2021	(VD) – water + ethane Mixture 1:1 (II)

14:15	Cryo-on

15:25	Laser On

15:25	T = 20.8	P = 5 * 10-10

15:30 MCT detector cooled

16:26 P = 5 e-10 T = 20.4K

## Background scan(s) #1

11:35	BG20210324_01

512 scans	res = 1 cm-1	signal = 3.55
               p = 5 * 10-10 mbar          T = 20.4K

## Deposition #4: C2H6 - ASW @ 20.5 K

17:15 	20 min @ 1x10-7 mbar C2H6 – ASW 1:1

- Initial Temperature: 20.4 K

- Initial pressure: 5*10-10 mbar

- Initial gas cell pressure: 0.993

- Deposition pressure: 1x10-7

- Laser signal: 311.2 mV

- Deposition time: 20 min

- final gas cell pressure = 0.944 Torr

17:32	laser Off / Head rotated

17:34	C2H6_ASW_2021_03_24_0001 (bg1)

512 scans	1 cm-1 res	signal 3.94
p = 3 x 10-9 mbar	T = 20.5 K

Macro 80 – exit at loop 73

25/03/2021

10 :37 T = 20.3K	P = 5 * 10e-10

10 :47 MCT Detector cooled

11:00	C2H6_ASW_2021_03_24_0074 (bg1)

512 scans	1 cm-1 res	signal 3.88
p = 5 x 10-10 mbar	T = 20.2 K

11:22	C2H6_ASW_2021_03_24_0075 (bg1)

512 scans	1 cm-1 res	signal 
p = 5 x 10-10 mbar	T = 20.2 K

11:40	C2H6_ASW_2021_03_24_0076 (bg1)

512 scans	1 cm-1 res	signal 3.92
p = 5 x 10-10 mbar	T = 20.2 K

12 :00	Macro 20 (up to scan 96)

16:45	C2H6_ASW_2021_03_24_0097 (bg1)

512 scans	1 cm-1 res	signal 3.89
p = 5 x 10-10 mbar	T = 20.2 K

17 :00	Macro 40

26/03/2021

11 :07	MCT Detector cooled

12 :54	T = 20.2	P = 5 *10-10

13:00	C2H6_ASW_2021_03_24_0138 (bg1)

512 scans	1 cm-1 res	signal 3.80
p = 5 x 10-10 mbar	T = 20.2 K

13:18	C2H6_ASW_2021_03_24_0139 (bg1)

512 scans	1 cm-1 res	signal 3.97
p = 5 x 10-10 mbar	T = 20.2 K

## Annealing to 130 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 14 :00 | _0140 | Warm-up | 3.97 |

14:18	C2H6_ASW_2021_03_24_0141 (bg1)

512 scans	1 cm-1 res	signal 3.67
p = 3.3 x 10-8 mbar	T = 129 K

14:34	C2H6_ASW_2021_03_24_0142 (bg1)

512 scans	1 cm-1 res	signal 3.67
p = 1.1 x 10-8 mbar	T = 129.9 K

14:49	C2H6_ASW_2021_03_24_0143 (bg1)

512 scans	1 cm-1 res	signal 3.67
p = 8 x 10-9 mbar	T = 129.9 K

15 :05

+ Macro 40 scans

27/03/2021

10 :44 T = 130k	P = 4.6 *10e-9

10:55 MCT Detector cooled

11:20	C2H6_ASW_2021_03_24_0184 (bg1)

512 scans	1 cm-1 res	signal 3.59
p = 4.5 x 10-9 mbar	T = 130 K

11:35	C2H6_ASW_2021_03_24_0185 (bg1)

512 scans	1 cm-1 res	signal 3.61
p = 4.5 x 10-9 mbar	T = 130 K

11:50	C2H6_ASW_2021_03_24_0186 (bg1)

512 scans	1 cm-1 res	signal 3.61
p = 4.5 x 10-9 mbar	T = 130 K

+  Macro 4 scans (up to 190)

## Annealing to 135 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 13:00 | _0191 | Warm-up | 3.58 |

+ Macro 3 scan

## Annealing to 140 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 14:00 | _0195 | Warm-up | 3.57 |

+ Macro 3 scan

## Annealing to 150 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 15:00 | _0199 | Warm-up | 3.55 |

+ Macro 20 scan (exit loop 9)

17 :30 Cryo off Heater off

# Friday 16th July 2021	(VD) – water + ethane 1:1 ratio

11:00	Cryo-on

11:00	Laser On

15:21	T = 19.9 K P = 1.1* 10-9 mbar

14:45 MCT detector cooled

## Background scan(s) #1

15:35	BG20210716_01

512 scans	res = 1 cm-1	signal = 3.67
               p = 1.1 * 10-9 mbar          T = 19.9K

## Deposition #: C2H6 – ASW 1:1 @ 20.5 K

16:00 	20 min @ 1x10-7 mbar C2H6 – ASW 1:1

- Initial Temperature: 19.9 K

- Initial pressure: 1*05 * -9 mbar

- Initial gas cell pressure: 1.067

- Deposition pressure: 1x10-7

- Laser signal: 327 mV

- Deposition time: 20 min

- final gas cell pressure = 1.018 Torr

16:20	laser Off / Head rotated

16:22	C2H6_ASW_2021_07_16_0001 (bg1)

512 scans	1 cm-1 res	signal 4.11
p =5 x 10-9 mbar	T = 19.9 K

16 :40 Macro 60 scans up to scan 61

17/07 13 :13 MCT Detector cooled

13:25	C2H6_ASW_2021_07_16_0062 (bg1)

512 scans	1 cm-1 res	signal 4.02
p =7 x 10-10 mbar	T = 19.9 K

13:40	C2H6_ASW_2021_07_16_0063 (bg1)

512 scans	1 cm-1 res	signal 4.11
p =7 x 10-10 mbar	T = 19.9 K

14 :00 Macro 40 up to scan 103

18/07/2021

15 :40 MCT Detector cooled

16:00	C2H6_ASW_2021_07_16_0104 (bg1)

512 scans	1 cm-1 res	signal 4.02
p =5.7 x 10-10 mbar	T = 19.8 K

16 :15 Macro 60 scans up to scan 164

19/07/2021

10 :43 MCT Detector cooled`

11:15	C2H6_ASW_2021_07_16_0165 (bg1)

512 scans	1 cm-1 res	signal 4.11
p =5 x 10-10 mbar	T = 19.6 K

11:30	C2H6_ASW_2021_07_16_0166 (bg1)

512 scans	1 cm-1 res	signal 4.21
p =5 x 10-10 mbar	T = 19.6 K

11:45	C2H6_ASW_2021_07_16_0167 (bg1)

512 scans	1 cm-1 res	signal 4.22
p =5 x 10-10 mbar	T = 19.6 K

12 :00 Macro 60 scans EXIT AT LOOP 30 up to scan 197

19:04 MCT Detector cooled

19:15	C2H6_ASW_2021_07_16_0198 (bg1)

512 scans	1 cm-1 res	signal 4.07
p =5 x 10-10 mbar	T = 19.6 K

19 :30 Macro 70 scans exit at loop 63 (10 :05) up to scan 261

20/07

10:30 MCT Detector cooled

10:45	C2H6_ASW_2021_07_16_0262 (bg1)

512 scans	1 cm-1 res	signal 3.93
p =5 x 10-10 mbar	T = 19.6 K

11 :00 Macro 60 scans up to scan 322

21/07/2021

16 :40 MCT Detector cooled

17:00	C2H6_ASW_2021_07_16_0323 (bg1)

512 scans	1 cm-1 res	signal 3.96
p =5 x 10-10 mbar	T = 19.6 K

17 :15 Macro 7 scan up to scan 330

## Annealing to 60 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 19 :00 | _0331 | Warm-up | 4.13 |

+ Macro 60 scans up to scan 391

22/07/2021

14:10 MCT Detector cooled

14:30	C2H6_ASW_2021_07_16_0392 (bg1)

512 scans	1 cm-1 res	signal 3.88
p =2.3 x 10-8 mbar	T = 60 K

14:45	C2H6_ASW_2021_07_16_0393 (bg1)

512 scans	1 cm-1 res	signal 4.11
p =2.1 x 10-8 mbar	T = 60 K

15 :00 Macro 60 scans up to scan 453

23/07/2021

11 :05 P = 5.8 * 10-9 T = 60K

11 :15 MCT Detector cooled

11:30	C2H6_ASW_2021_07_16_0454 (bg1)

512 scans	1 cm-1 res	signal 3.98
p =5.6 x 10-9 mbar	T = 60 K

11:45	C2H6_ASW_2021_07_16_0455 (bg1)

512 scans	1 cm-1 res	signal 3.98
p =5.6 x 10-9 mbar	T = 60 K

12 :00 Macro 60 scans up to scan 515

24/07/2021

13 :10 P = 2.4 * 10-9 T = 60K

13 :12 MCT Detector cooled

13:30	C2H6_ASW_2021_07_16_0516 (bg1)

512 scans	1 cm-1 res	signal 3.93
p =2.4 x 10-9 mbar	T = 60 K

13:45	C2H6_ASW_2021_07_16_0517 (bg1)

512 scans	1 cm-1 res	signal 3.98
p =2.4 x 10-9 mbar	T = 60 K

14 :00 Macro 30 scans exit at loop 17 up to scan 534

## Annealing to 70 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 18 :00 | _0535 | Warm-up | 3.89 |

18:15	C2H6_ASW_2021_07_16_0536 (bg1)

512 scans	1 cm-1 res	signal 3.86
p =3. x 10-7 mbar	T = 69.8 K

19:00	C2H6_ASW_2021_07_16_0537 (bg1)

512 scans	1 cm-1 res	signal 3.84
p =7.1 x 10-8 mbar	T = 69.9 K

19:15	C2H6_ASW_2021_07_16_0538 (bg1)

512 scans	1 cm-1 res	signal 3.83
p =5.8 x 10-8 mbar	T = 70.0 K

19:30	C2H6_ASW_2021_07_16_0539 (bg1)

512 scans	1 cm-1 res	signal 3.83
p =4.2 x 10-8 mbar	T = 70.0 K

## Annealing to 135 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 19 :45 | _0540 | Warm-up | 3.83 |

20:00	C2H6_ASW_2021_07_16_0541 (bg1)

512 scans	1 cm-1 res	signal 3.83
p =1.5 x 10-7 mbar	T = 133.4 K

20:15	C2H6_ASW_2021_07_16_0542 (bg1)

512 scans	1 cm-1 res	signal 3.83
p =5.6 x 10-8 mbar	T = 135K

20 :32 MCT Detector cooled

20:45	C2H6_ASW_2021_07_16_0543 (bg1)

512 scans	1 cm-1 res	signal 3.70
p =3.5 x 10-8 mbar	T = 135K

21 :00 Macro 80 scans exit at loop 65 up to scan 608

25/07/2021

12:12 MCT Detector cooled

12:30	C2H6_ASW_2021_07_16_0609 (bg1)

512 scans	1 cm-1 res	signal 3.70
p =1.5 x 10-8 mbar	T = 135K

12:45	C2H6_ASW_2021_07_16_0610 (bg1)

512 scans	1 cm-1 res	signal 3.70
p =1.4 x 10-8 mbar	T = 135K

13 :00 Macro 80 scans exit at loop 23 up to scan 633

18:26 MCT Detector cooled

18:45	C2H6_ASW_2021_07_16_0634 (bg1)

512 scans	1 cm-1 res	signal 3.66
p =7.6 x 10-9 mbar	T = 135K

19 :00 Macro 80 scans exit at loop 67 up to scan 701

26/07/2021

10:39 MCT Detector cooled

11:00	C2H6_ASW_2021_07_16_0702 (bg1)

512 scans	1 cm-1 res	signal 3.73
p =5.5 x 10-9 mbar	T = 135K

11:15	C2H6_ASW_2021_07_16_0703 (bg1)

512 scans	1 cm-1 res	signal 3.73
p =5.5 x 10-9 mbar	T = 135K

11:30	C2H6_ASW_2021_07_16_0704 (bg1)

512 scans	1 cm-1 res	signal 3.73
p =5.6 x 10-9 mbar	T = 135K

11:45	C2H6_ASW_2021_07_16_0705 (bg1)

512 scans	1 cm-1 res	signal 3.72
p =5.6 x 10-9 mbar	T = 135K

12 :00 Macro 10 scans up to scan 715

14:30	C2H6_ASW_2021_07_16_0716 (bg1)

512 scans	1 cm-1 res	signal 3.71
p =5.5 x 10-9 mbar	T = 135K

## Annealing to 140 K

512 scans; 1 cm-1 res

| time | File | Type | signal |
| --- | --- | --- | --- |
| 14 :45 | _0717 | Warm-up | 3.70 |

15:00	C2H6_ASW_2021_07_16_0718 (bg1)

512 scans	1 cm-1 res	signal 3.71
p =1.8 x 10-8 mbar	T = 139.9K

Macro 30 scans up to scan 748

27/07/2021

11 :37 MCT Detector cooled

12:03	C2H6_ASW_2021_07_16_0749 (bg1)

512 scans	1 cm-1 res	signal 3.56
p =1.2 x 10-9 mbar	T = 140K

12:30	C2H6_ASW_2021_07_16_0750 (bg1)

512 scans	1 cm-1 res	signal 3.90
p =1.2 x 10-9 mbar	T = 140K

12:45	C2H6_ASW_2021_07_16_0751 (bg1)

512 scans	1 cm-1 res	signal 3.90
p =1.2 x 10-9 mbar	T = 140K

13 :00 Macro 30 scans up to scan 781

28/07/2021

11 :00 MCT Detector cooled

11:30	C2H6_ASW_2021_07_16_0782 (bg1)

512 scans	1 cm-1 res	signal 3.82
p =9.0 x 10-10 mbar	T = 140

12 :23 Cryo off Heater manual 0

# Monday 16th August 2021	(VD) – ethane +  water + ethane (multi deposition)

11:05	Cryo-on

11:05	Laser On

12:00	MCT Detector cooled

13:20: T = 19.9 K ; P = 5 * 10-10 mbar

## Background scan(s) #1

13:30	BG20210816_01

512 scans	res = 1 cm-1	signal = 3.67
               p = 5 * 10-10 mbar          T = 19.9K

## Deposition 1#: C2H6 @ 19.9 K

14:00 	10 min @ 1x10-7 mbar C2H6

- Initial Temperature: 19.9 K

- Initial pressure: 5.0 * 10 -10 mbar

- Initial gas cell pressure: 1.035 (Ethane)

- Deposition pressure: 1x10-7

- Laser signal: 326.2 mV

- Deposition time: 10 min

- final gas cell pressure = 1.021 Torr

14:12	laser Head rotated

Gas cell valve open for pumping

14:15	C2H6_ASW_2021_08_16_0001 (bg1)

512 scans	1 cm-1 res	signal 3.74
p =5.0 x 10-10 mbar	T = 19.9 K

14:30	C2H6_ASW_2021_08_16_0002 (bg1)

512 scans	1 cm-1 res	signal 3.73
p =5.0 x 10-10 mbar	T = 19.9 K

15:00	BG20210816_02

512 scans	res = 1 cm-1	signal = 3.67
               p = 5 * 10-10 mbar          T = 19.9K

## Deposition 2#: H2O @ 19.9 K

15:30 	20 min @ 1x10-7 mbar ASW

- Initial Temperature: 19.9 K

- Initial pressure: 5.0 * 10 -10 mbar

- Initial gas cell pressure: 0.734 (Water)

- Deposition pressure: 1x10-7

- Laser signal: 324.6 mV

- Deposition time: 20 min

- final gas cell pressure = 0.610 Torr

15:52	laser Head rotated

Gas cell valve open for pumping

16:00	C2H6_ASW_2021_08_16_0003 (bg2)

512 scans	1 cm-1 res	signal 3.72
p =mbar	T = 19.9 K

16:15	C2H6_ASW_2021_08_16_0004 (bg1)

512 scans	1 cm-1 res	signal 3.72
p =2.0 x 10-9 mbar	T = 19.9 K

16:30	BG20210816_03

512 scans	res = 1 cm-1	signal = 3.72
               p = 1.5 * 10-9 mbar          T = 19.9K

## Deposition #: C2H6 @ 20 K

15:30 	10 min @ 1x10-7 mbar C2H6

- Initial Temperature: 20 K

- Initial pressure: 1.1 * 10 -9 mbar

- Initial gas cell pressure: 1.274 (C2H6)

- Deposition pressure: 1x10-7

- Laser signal: 322.7 mV

- Deposition time: 10 min

- final gas cell pressure = 1.259 Torr

17:12	laser Head rotated

Gas cell valve open for pumping

LASER OFF

17:15	C2H6_ASW_2021_08_16_0005 (bg3)

512 scans	1 cm-1 res	signal 3.72
p = 1 * 10-9 mbar	T = 19.9 K

17:30	C2H6_ASW_2021_08_16_0006 (bg1)

512 scans	1 cm-1 res	signal 3.71
p =mbar	T = 19.9 K

17:45	C2H6_ASW_2021_08_16_0007 (bg1)

512 scans	1 cm-1 res	signal 3.71
p = 7.5 * 10-10mbar	T = 19.9 K

18 :00 Macro 20 scans

Exit at loop 11 up to scan 18

20:41 MCT Detector cooled

21:00	C2H6_ASW_2021_08_16_0019 (bg1)

512 scans	1 cm-1 res	signal 3.70
p = 5 * 10-10mbar	T = 19.9 K

21 :15 Macro 80 scan exit at loop 60 up to scan 79

17/08/2021

11:24 MCT Detector cooled

12:00	C2H6_ASW_2021_08_16_0080 (bg1)

512 scans	1 cm-1 res	signal 3.72
p = 5 * 10-10mbar	T = 19.7 K

12 :15 Macro 20 scans (loop 12 – too many bad scans – data collection has stopped 15 :44)

Exit at loop 19 (up to scan)

17:30	C2H6_ASW_2021_08_16_0099 (bg1)

512 scans	1 cm-1 res	signal 3.72
p = 5 * 10-10mbar	T = 19.7 K

Messed up with the ramp … (went to 40 and the down)

17:45	C2H6_ASW_2021_08_16_0100 (bg1)

512 scans	1 cm-1 res	signal 3.72
p =	T = 21.0 K

18:00	C2H6_ASW_2021_08_16_0101 (bg1)

512 scans	1 cm-1 res	signal 3.65
p = 6.8 * 10-10mbar	T = 19.8 K

18 :17 MCT Detector cooled

18:30	C2H6_ASW_2021_08_16_0102 (bg1)

512 scans	1 cm-1 res	signal 3.65
p = 6.8 * 10-10mbar	T = 19.8 K

Macro 60 scans (exit at loop 59 up to scan 161)

18/08/2021

9 :00 MCT Detector cooled

09:45	C2H6_ASW_2021_08_16_0162 (bg1)

512 scans	1 cm-1 res	signal 3.65
p = 5 * 10-10mbar	T = 19.8 K

10 :00 Macro 20 scans (exit at loop 6 – problem !)

12:29Restart omnic

12:30	C2H6_ASW_2021_08_16_0168 (bg1)

512 scans	1 cm-1 res	signal 3.50
p = 5 * 10-10mbar	T = 19.7 K

13:45	C2H6_ASW_2021_08_16_0169 (bg1)

512 scans	1 cm-1 res	signal 3.65
p = 5 * 10-10mbar	T = 19.7 K

19/08/2021

12 :30 MCT Detector cooled

13:30	C2H6_ASW_2021_08_16_0170 (bg1)

512 scans	1 cm-1 res	signal 3.73
p = 5 * 10-10mbar	T = 19.7 K

13 :45 Macro 20 scans

18 :30 MCT Detector cooled

18:45	C2H6_ASW_2021_08_16_0191 (bg1)

512 scans	1 cm-1 res	signal 3.64
p = 5 * 10-10mbar	T = 19.7 K

19 :00 Macro 80 scans (exit at loop 66 up to scan 257)

20/08/2021

11:32 MCT Detector cooled

11:45	C2H6_ASW_2021_08_16_0258 (bg1)

512 scans	1 cm-1 res	signal 3.73
p = 5 * 10-10mbar	T = 19.6 K

12:00	C2H6_ASW_2021_08_16_0259 (bg1)

512 scans	1 cm-1 res	signal 3.76
p = 5 * 10-10mbar	T = 19.6 K

12 :20 Macro 10 scan

15:00	C2H6_ASW_2021_08_16_0270 (bg1)

512 scans	1 cm-1 res	signal 3.67
p = 5 * 10-10mbar	T = 19.6 K

15:15	C2H6_ASW_2021_08_16_0271 (bg1)

512 scans	1 cm-1 res	signal 3.66
p = 5 * 10-10mbar	T = 19.6 K

17:00	C2H6_ASW_2021_08_16_0272 (bg1)

512 scans	1 cm-1 res	signal 3.24
p = 5 * 10-10mbar	T = 19.6 K

22/08/2021

(computer has restarted – had to recover labbok (no loss of information) – Omnic restarted)

12 :20 MCT Detector cooled

14:00	C2H6_ASW_2021_08_16_0273 (bg1)

512 scans	1 cm-1 res	signal 3.71
p = 5 * 10-10mbar	T = 19.6 K

14:30	C2H6_ASW_2021_08_16_0274 (bg1)

512 scans	1 cm-1 res	signal 3.71
p = 5 * 10-10mbar	T = 19.6 K

15 :00 Macro 10 scans (exit at loop 5)

16:30	C2H6_ASW_2021_08_16_0280 (bg1)

512 scans	1 cm-1 res	signal 3.71
p = 5 * 10-10mbar	T = 19.6 K

## Annealing to 30 K

16:45	C2H6_ASW_2021_08_16_0280 (bg1)

512 scans	1 cm-1 res	signal 3.66
p = 5 * 10-10mbar	T = 19.6 K

17:00	C2H6_ASW_2021_08_16_0281 (bg1)

512 scans	1 cm-1 res	signal 3.66
p = 7.7 * 10-10mbar	T = 29.9 K

17:30	C2H6_ASW_2021_08_16_0282 (bg1)

512 scans	1 cm-1 res	signal 3.66
p = 7.7 * 10-10mbar	T = 29.9 K

## Annealing to 40 K

18:00	C2H6_ASW_2021_08_16_0283 (bg1)

512 scans	1 cm-1 res	signal 3.66
p = 5 * 10-10mbar	T = 19.6 K

18:15	C2H6_ASW_2021_08_16_0284 (bg1)

512 scans	1 cm-1 res	signal 3.66
p = 5 * 10-10mbar	T = 19.6 K

Time out problem – go home

23/08/2021

10 :55 MCT Detector cooled

11:30	C2H6_ASW_2021_08_16_0285 (bg1)

512 scans	1 cm-1 res	signal 3.63
p = 5 * 10-10mbar	T = 40 K

Lots of time out, bad scans … Why ?

12:00	C2H6_ASW_2021_08_16_0286 (bg1)

512 scans	1 cm-1 res	signal 3.63
p = 5 * 10-10mbar	T = 40 K

Computer turned off (on its own)

14:00	C2H6_ASW_2021_08_16_0287 (bg1)

512 scans	1 cm-1 res	signal 3.60
p = 5 * 10-10mbar	T = 40 K

15:30	C2H6_ASW_2021_08_16_0288 (bg1)

512 scans	1 cm-1 res	signal 3.60
p = 5 * 10-10mbar	T = 40 K

27/08/2021

12 :00 MCT Detector cooled

14:30	C2H6_ASW_2021_08_16_0289 (bg1)

512 scans	1 cm-1 res	signal 3.60
p = 5 * 10-10mbar	T = 40 K

13/09

# Wednesday 15th Sep 2021	(VD) – ethane only (clean dep)

10:00	Cryo-on

10:00	Laser On

10:45	MCT Detector cooled

11:05: T = 67.5 K ; P = 5.8 * 10-9 mbar

## Background scan(s) #1

14:00	BG20210915_01

512 scans	res = 1 cm-1	signal = 3.45
               p = 4.1 * 10-9 mbar          T = 20K

## Deposition #1: C2H6 @ 20.0 K

17:45 Head rotated

18:00 	20 min @ 1x10-7 mbar C2H6

- Initial Temperature: 20.0 K

- Initial pressure: 3.4*10-9 mbar

- Initial gas cell pressure: (before introducing ethane reading = 0.057) – 1.046

- Deposition pressure: 1x10-7

- Laser signal: 305.1 mV

- Deposition time: 20 min

- pressure after deposition 3.3 * 10-9 mbar

- final gas cell pressure = 1.019 Torr

18:22	laser Off / Head rotated

18:32	C2H6_2021_09_15_0001

512 scans	1 cm-1 res	signal 3.60
p = 3.2 x 10-9 mbar	T = 20.0 K

Problem with spectro 	…

19:40	C2H6_2021_09_15_0002

512 scans	1 cm-1 res	signal 3.70
p = 3.2 x 10-9 mbar	T = 20.0 K

20:05	C2H6_2021_09_15_0003

512 scans	1 cm-1 res	signal 3.68
p = 3.2 x 10-9 mbar	T = 20.0 K

16/09

10 :15 MCT Detector cooled

10:45	C2H6_2021_09_15_0004

512 scans	1 cm-1 res	signal 3.45
p = 2.7 x 10-9 mbar	T = 20.0 K

11:00	C2H6_2021_09_15_0005

512 scans	1 cm-1 res	signal 3.63
p = 2.7 x 10-9 mbar	T = 20.0 K

11:15	C2H6_2021_09_15_0006

512 scans	1 cm-1 res	signal 3.63
p = 2.7 x 10-9 mbar	T = 20.0 K

11:30	C2H6_2021_09_15_0007

512 scans	1 cm-1 res	signal 3.63
p = 2.6 x 10-9 mbar	T = 20.0 K

11:45	C2H6_2021_09_15_0008

512 scans	1 cm-1 res	signal 3.67
p = 2.6 x 10-9 mbar	T = 20.0 K

12:00	C2H6_2021_09_15_0009

512 scans	1 cm-1 res	signal 3.67
p = 2.6 x 10-9 mbar	T = 20.0 K

13:00	C2H6_2021_09_15_0010

512 scans	1 cm-1 res	signal 3.67
p = 2.6 x 10-9 mbar	T = 20.0 K

## Annealing to 30 K

14:00	C2H6_2021_09_15_0011

512 scans	1 cm-1 res	signal 3.66
p = 2.6 x 10-9 mbar	T = 20.0 K

14:15	C2H6_2021_09_15_0012

512 scans	1 cm-1 res	signal 3.66
p = 2.7 x 10-9 mbar	T = 30.0 K

14:30	C2H6_2021_09_15_0013

512 scans	1 cm-1 res	signal 3.66
p = 2.6 x 10-9 mbar	T = 30.0 K

14:45	C2H6_2021_09_15_0014

512 scans	1 cm-1 res	signal 3.66
p = 2.5 x 10-9 mbar	T = 30.0 K

## Annealing to 40 K

15:00	C2H6_2021_09_15_0015

512 scans	1 cm-1 res	signal 3.66
p = 2.5 x 10-9 mbar	T = 30.0 K

15:15	C2H6_2021_09_15_0016

512 scans	1 cm-1 res	signal 3.66
p = 4.9 x 10-9 mbar	T = 39.8 K

15:30	C2H6_2021_09_15_0017

512 scans	1 cm-1 res	signal 3.66
p = 3.3 x 10-9 mbar	T = 40K

15:45	C2H6_2021_09_15_0018

512 scans	1 cm-1 res	signal 3.66
p = 3.3 x 10-9 mbar	T = 40K

Peak at 3258 ?

## Annealing to 50 K

16:00	C2H6_2021_09_15_0019

512 scans	1 cm-1 res	signal 3.67
p = 2.9 x 10-9 mbar	T = 40K

16:15	C2H6_2021_09_15_0020

512 scans	1 cm-1 res	signal 3.67
p =	T = 50K

16:30	C2H6_2021_09_15_0021

512 scans	1 cm-1 res	signal 3.67
p = 2.9 x 10-9 mbar	T = 50K

16:45	C2H6_2021_09_15_0022

512 scans	1 cm-1 res	signal 3.68
p = 2.8 x 10-9 mbar	T = 50K

## Annealing to 55 K

17:00	C2H6_2021_09_15_0023

512 scans	1 cm-1 res	signal 3.68
p = 2.8 x 10-9 mbar	T = 50K

17:15	C2H6_2021_09_15_0024

512 scans	1 cm-1 res	signal 3.68
p = 3.4 x 10-9 mbar	T = 55K

17 :30 Macro 20 scans up to scan 44

17/09/2021

11 :45 MCT Detector cooled

13:15	C2H6_2021_09_15_0045

512 scans	1 cm-1 res	signal 3.51
p = 2.5 x 10-9 mbar	T = 55K

13:30	C2H6_2021_09_15_0046

512 scans	1 cm-1 res	signal 3.74
p = 2.5 x 10-9 mbar	T = 55K

## Annealing to 60 K

13:45	C2H6_2021_09_15_0047

512 scans	1 cm-1 res	signal 3.74
p = 2.5 x 10-9 mbar	T = 55K

14:00	C2H6_2021_09_15_0048

512 scans	1 cm-1 res	signal 3.74
p = 2.5 x 10-8 mbar	T = 60K

14 :15 Macro 20 scans Exit at loop 5

## Annealing to 65 K

15:30	C2H6_2021_09_15_0054

512 scans	1 cm-1 res	signal 3.71
p = 3.3 x 10-9 mbar	T = 60.6K

15:45	C2H6_2021_09_15_0055

512 scans	1 cm-1 res	signal 3.68
p = 8.1 x 10-7 mbar	T = 65K

16 :00 Macro 20 scans exit at loop 7 (up to scan 61)

17:33 Cryo off

# Saturday 18th Sep 2021	(VD) – ethane at 40K dep

14:40 T = 278.6 P = 3.34 * 10-8

14:41	Cryo-on

14:41	Laser On

15:30	MCT Detector cooled

16:05 T = 20.2

P = 3.3 * 10-9

Heater set up 40K

16:30

## Background scan(s) #1

16:35	BG20210918_01

512 scans	res = 1 cm-1	signal = 3.50
               p = 3.6 * 10-9 mbar          T = 40.0

## Deposition #2: C2H6 @ 40 K

16:50 Head rotated

17:00 	20 min @ 1x10-7 mbar C2H6

- Initial Temperature: 40.0 K

- Initial pressure: 3.3*10-9 mbar

- Initial gas cell pressure: 1.018 (same as previous deposition)

- Deposition pressure: 1x10-7

- Laser signal: 325.1 mV

- Deposition time: 20 min

- pressure after deposition 3.3 * 10-9 mbar

- final gas cell pressure = 0.993 Torr

17:20	laser Off / Head rotated

17:25	C2H6_2021_09_18_0001

512 scans	1 cm-1 res	signal 3.68
p = 3.1 x 10-9 mbar	T = 40.0K

17 :40 Macro 20 scans (exit6 at loop 7 up to scan )

## Annealing to 50 K

19:20	C2H6_2021_09_18_0009

512 scans	1 cm-1 res	signal 3.60
p = 2.9 x 10-9 mbar	T = 40.0 K

19:35	C2H6_2021_09_18_0010

512 scans	1 cm-1 res	signal 3.59
p = 2.9 x 10-9 mbar	T = 50.0 K

19:50	C2H6_2021_09_18_0011

512 scans	1 cm-1 res	signal 3.59
p = 2.9 x 10-9 mbar	T = 50.0 K

## Annealing to 65 K

20:05	C2H6_2021_09_18_0012

512 scans	1 cm-1 res	signal 3.60
p = 2.9 x 10-9 mbar	T = 50.0 K

20:20 Macro 20 scans (exit at loop7)

22:00 cryo off – heater off

# Sunday 19th Sep 2021	(VD) – ethane at 20K for 30 min dep

14:40 T = 276.7 P = 2.7 * 10-8

12:42 T set up to 300K

12:52 T = 300.4 K; P = 9.6 * 10-8 mbar (heater stopped)

12:55	Cryo-on

12:55	Laser On

13:23	MCT Detector cooled

14:57: T = 19.9 P = 3.7 e-9

## Background scan(s) #1

15:00	BG20210919_01

512 scans	res = 1 cm-1	signal = 3.40
               p = 3.6 * 10-9 mbar          T = 19.9

## Deposition #3: C2H6 @ 19.9 K

15:16 Head rotated

15:20 	30 min @ 1x10-7 mbar C2H6

- Initial Temperature: 19.9 K

- Initial pressure: 3.4*10-9 mbar

- Initial gas cell pressure: 0.993

- Deposition pressure: 1x10-7

- Laser signal: 325.6 mV

- Deposition time: 30 min

- pressure after deposition 3.0 * 10-9 mbar

- final gas cell pressure = 0.955 Torr

15:50	laser Off / Head rotated

16:00	C2H6_2021_09_19_0001

512 scans	1 cm-1 res	signal 3.63
p = 2.8 x 10-9 mbar	T = 19.9K

16:15	C2H6_2021_09_19_0002

512 scans	1 cm-1 res	signal 3.63
p = 2.8 x 10-9 mbar	T = 19.9K

## Annealing to 40 K

16:30	C2H6_2021_09_15_0003

512 scans	1 cm-1 res	signal 3.61
p = 2.8 x 10-9 mbar	T = 19.9

16:45	C2H6_2021_09_15_0004

512 scans	1 cm-1 res	signal 3.61
p = 2.8 x 10-9 mbar	T = 40

17:00	C2H6_2021_09_15_0005

512 scans	1 cm-1 res	signal 3.60
p = 2.8 x 10-9 mbar	T = 40

## Annealing to 65 K

17:15	C2H6_2021_09_15_0006

512 scans	1 cm-1 res	signal 3.60
p = 2.8 x 10-9 mbar	T = 40

17 :30 Macro 20 scan (exit at loop 13)

20:30 cryo off, heater off

# Monday 20th Sep 2021	(VD) – ethane on top ASW 20K

10:12 T = 276 K P = 2.4 * 10-8

10:15	Cryo-on

10:15	Laser On

11:10	MCT Detector cooled

12:27 T = 19.7K P = 2.3 * 10 e-9

## Background scan(s) #1

12:40	BG20210920_01

512 scans	res = 1 cm-1	signal = 3.72
               p = 2.2 * 10-9 mbar          T = 19.7

17 :00 Head rotated (laser moved so put it back in place)

## Deposition 1: H2O @ 20K

13:15 	20 min @ 1x10-7 mbar H2O

- Initial Temperature: 19.7K

- Initial pressure: 2.1 * 10-9 mbar

- Initial gas cell pressure: 7.73 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  326.0 mV

- Deposition time: 20 min

- pressure after deposition 9.3 e-9

- final gas cell pressure = 6.63 Torr

13:36	Head rotated

13:40	C2H6_ASW_2021_09_20_0001

512 scans	1 cm-1 res	signal 3.77
p = 5 x 10-9 mbar	T = 19.7K

14:30	C2H6_ASW_2021_09_20_0002

512 scans	1 cm-1 res	signal 3.75
p = 2.2 x 10-9 mbar	T = 19.7

14:45	C2H6_ASW_2021_09_20_0003

512 scans	1 cm-1 res	signal 3.75
p = 2.1 x 10-9 mbar	T = 19.7K

15:00	BG20210920_02

512 scans	res = 1 cm-1	signal = 3.75
               p = 1.9 * 10-9 mbar          T = 19.7

## Deposition 2: C2H6 @ 19.7 K

15:30 	20 min @ 1x10-7 mbar C2H6

- Initial Temperature: 19.7 K

- Initial pressure: 1.8*10-9 mbar

- Initial gas cell pressure: 1.232

- Deposition pressure: 1x10-7

- Laser signal: 324.7 mV

- Deposition time: 20 min

- pressure after deposition 3 * 10-9 mbar

- final gas cell pressure = 1.167 Torr

15:52	laser Off / Head rotated

Problem with spectro (had to switch on/off)

16:07	C2H6_ASW_2021_09_20_0004 (bg1)

512 scans	1 cm-1 res	signal 3.81
p = 2.1 x 10-9 mbar	T = 19.7K

Very few ethane again (similar to 02/24) – also water contribution seems to decrease …

16:30	C2H6_ASW_2021_09_20_0005 (bg1)

512 scans	1 cm-1 res	signal 3.81
p = 1.9 x 10-9 mbar	T = 19.7K

16:45	C2H6_ASW_2021_09_20_0006 (bg1)

512 scans	1 cm-1 res	signal 3.81
p = 1.9 x 10-9 mbar	T = 19.7K

## Annealing to 40 K

17:00	C2H6_ASW_2021_09_20_0007

512 scans	1 cm-1 res	signal 3.82
p = 1.8 x 10-9 mbar	T = 19.7

17:15	C2H6_ASW_2021_09_20_0008

512 scans	1 cm-1 res	signal 3.78
p = 4.5 x 10-7 mbar	T = 39.7

17:30	C2H6_ASW_2021_09_20_0009

512 scans	1 cm-1 res	signal 3.77
p = 1.9 x 10-7 mbar	T = 40

17:45	C2H6_ASW_2021_09_20_0010

512 scans	1 cm-1 res	signal 3.76
p = 8.6 x 10-8 mbar	T = 40

## Annealing to 65 K

18:00	C2H6_ASW_2021_09_20_0011

512 scans	1 cm-1 res	signal 3.75
p = 7.7 x 10-8 mbar	T = 40

18:15	C2H6_ASW_2021_09_20_0012

512 scans	1 cm-1 res	signal 3.73
p = 3.9 x 10-8 mbar	T = 65

18:30	C2H6_ASW_2021_09_20_0013

512 scans	1 cm-1 res	signal 3.73
p = 1.5 x 10-8 mbar	T = 65

18:45	C2H6_ASW_2021_09_20_0014

512 scans	1 cm-1 res	signal 3.72
p = 1.0 x 10-8 mbar	T = 65

## Annealing to 90 K

19:00	C2H6_ASW_2021_09_20_0015

512 scans	1 cm-1 res	signal 3.72
p = 7.7 x 10-8 mbar	T = 40

19 :15 Macro 3 scan

## Annealing to 120 K

20:00	C2H6_ASW_2021_09_20_0019

512 scans	1 cm-1 res	signal 3.71
p = 5.3 x 10-9 mbar	T = 90

20 :15 Macro 3 scan

## Annealing to 135 K

21:00	C2H6_ASW_2021_09_20_0023

512 scans	1 cm-1 res	signal 3.71
p = 6.0 x 10-9 mbar	T = 120

21 :15 Macro 2 scans

21 :48 MCT Detector cooled

22:15	C2H6_ASW_2021_09_20_0026

512 scans	1 cm-1 res	signal 3.71
p = 2.9 x 10-8 mbar	T = 135

22 :30 Macro 80 scan exit at loop 58 up to scan 84

21/09/2021

12:06 MCT Detector cooled

12:28

12:30	C2H6_ASW_2021_09_20_0085

512 scans	1 cm-1 res	signal 3.71
p = 1.3 x 10-8 mbar	T = 135

12 :45 Macro 80 scans exit at loop 33 (I guess) up to scan 119

20:42 MCT Detector cooled

21:00	C2H6_ASW_2021_09_20_0120

512 scans	1 cm-1 res	signal 3.68
p = 1.0 x 10-8 mbar	T = 135

21 :15 Macro 80 exit at loop 63

22/09/2021

11:57 MCT Detector cooled

12 :15	C2H6_ASW_2021_09_20_0184

512 scans	1 cm-1 res	signal 3.68
p = 8.8 x 10-9 mbar	T = 134.9

Macro 80 scans exit at loop 21

## Annealing to 140 K

17:30	C2H6_ASW_2021_09_20_0206

512 scans	1 cm-1 res	signal 3.69
p = 8.6 x 10-9 mbar	T = 135K

17:45	C2H6_ASW_2021_09_20_0207

512 scans	1 cm-1 res	signal 3.69
p = 3.0 x 10-8 mbar	T = 140K

18:00	C2H6_ASW_2021_09_20_0208

512 scans	1 cm-1 res	signal 3.69
p = 3.1 x 10-8 mbar	T = 140K

18 :15 Heater off / cryo off

# Friday 23rd Sep 2021	(VD) – ethane on top ASW 20K  II

10:10 T = 277.5 K P = 2.7 * 10-8

10:11	Cryo-on

10:11	Laser On

11:45	MCT Detector cooled

11:45 T = 20K P = 2.6 * 10 e-9

12:23 T = 19.8	P = 2.3 * 10-9 mbar

## Background scan(s) #1

12:30	BG20210923_01

512 scans	res = 1 cm-1	signal = 3.69
               p = 2.3 * 10-9 mbar          T = 19.8

## Deposition 1: H2O @ 20K

13:05 	20 min @ 1x10-7 mbar H2O

- Initial Temperature: 19.7K

- Initial pressure: 2.2 * 10-9 mbar

- Initial gas cell pressure: 7.70 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  327.7 mV

- Deposition time: 20 min

- pressure after deposition 9.1 e-9

- final gas cell pressure = 6.54 Torr

13:26 Head rtotatexd

13:30	C2H6_ASW_2021_09_23_0001

512 scans	1 cm-1 res	signal 3.71
p = 5.0 x 10-9 mbar	T = 19.8K

13:50	C2H6_ASW_2021_09_23_0002

512 scans	1 cm-1 res	signal 3.70
p = 2.8 x 10-9 mbar	T = 19.8K

14:05	BG20210923_02

512 scans	res = 1 cm-1	signal = 3.68
p = 2.4 * 10-9 mbar          T = 19.8

14 :20 Head rotated

## Deposition 2: C2H6 @ 20K

14:30 	20 min @ 1x10-7 mbar H2O

- Initial Temperature: 19.8K

- Initial pressure: 2.1 * 10-9 mbar

- Initial gas cell pressure: 8.61 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  326.1 mV

- Deposition time: 20 min

- pressure after deposition 1.9 e-9

- final gas cell pressure = 8.35 Torr

14:50	 Head rotated

14:55	C2H6_ASW_2021_09_23_0003 (bg2)

512 scans	1 cm-1 res	signal 3.67
p = 1.8 x 10-9 mbar	T = 19.8K

15:10	C2H6_ASW_2021_09_23_0004 (bg1)

512 scans	1 cm-1 res	signal 3.67
p = 1.7 x 10-9 mbar	T = 19.8K

15:25	C2H6_ASW_2021_09_23_0005 (bg1)

512 scans	1 cm-1 res	signal 3.65
p = 1.7 x 10-9 mbar	T = 19.8K

15:40	C2H6_ASW_2021_09_23_0006 (bg1)

512 scans	1 cm-1 res	signal 3.65
p = 1.7 x 10-9 mbar	T = 19.8K

## Annealing to 40 K

16:00	C2H6_ASW_2021_09_23_0007

512 scans	1 cm-1 res	signal 3.63
p = 1.7 x 10-9 mbar	T = 19.8

Macro 3 scans

## Annealing to 65 K

17:00	C2H6_ASW_2021_09_23_0011

512 scans	1 cm-1 res	signal 3.64
p = 2 x 10-9 mbar	T = 40

Macro 20 scans exit at loop 10 up to scan 21

19:45	C2H6_ASW_2021_09_23_0022

512 scans	1 cm-1 res	signal 3.58
p = 5.2 x 10-8 mbar	T = 65

## Annealing to 90 K

20:00	C2H6_ASW_2021_09_23_0023

512 scans	1 cm-1 res	signal 3.58
p = 2 x 10-9 mbar	T = 40

20:15	C2H6_ASW_2021_09_23_0024

512 scans	1 cm-1 res	signal 3.58
p = 1.9 x 10-8 mbar	T = 89.7

20 :37 MCT detector cooled

20:45	C2H6_ASW_2021_09_23_0025

512 scans	1 cm-1 res	signal 3.56
p = 4.8 x 10-9 mbar	T = 90

21 :00 Macro 80 scans exit at loop 55

10:20	C2H6_ASW_2021_09_23_0081

512 scans	1 cm-1 res	signal 3.59
p = 1.6 x 10-9 mbar	T = 90

10 :38 MCT Detector cooled

11:00	C2H6_ASW_2021_09_23_0082

512 scans	1 cm-1 res	signal 3.58
p = 1.6 x 10-9 mbar	T = 90

## Annealing to 120 K

11:15	C2H6_ASW_2021_09_23_0083

512 scans	1 cm-1 res	signal 3.58
p = 1.5 x 10-9 mbar	T = 90

11 :30	Macro 8 scan

13:30	C2H6_ASW_2021_09_23_0092

512 scans	1 cm-1 res	signal 3.60
p = 2.4 x 10-9 mbar	T = 120

## Annealing to 135 K

13:45	C2H6_ASW_2021_09_23_0093

512 scans	1 cm-1 res	signal 3.61
p = 1.5 x 10-9 mbar	T = 120

14 :02	Macro 80 scan exit at loop 14 up to scan 107

17:27 MCT Detector cooled

17:45	C2H6_ASW_2021_09_23_0108

512 scans	1 cm-1 res	signal 3.63
p = 1.5 x 10-9 mbar	T = 135

Macro 80 scan up to loop 60 (scan)

25/09/2021

15:11 MCT Detector cooled

15:30	C2H6_ASW_2021_09_23_0167

512 scans	1 cm-1 res	signal 3.68
p = 8.7 x 10-9 mbar	T = 135

16:00	C2H6_ASW_2021_09_23_0168

512 scans	1 cm-1 res	signal 3.68
p = 8.6 x 10-9 mbar	T = 135

16:35	C2H6_ASW_2021_09_23_0169

512 scans	1 cm-1 res	signal 3.68
p = 8.5 x 10-9 mbar	T = 135

## Annealing to 140 K

17:00	C2H6_ASW_2021_09_23_0170

512 scans	1 cm-1 res	signal 3.71
p = 8.4 x 10-9 mbar	T = 135

17:25	C2H6_ASW_2021_09_23_0171

512 scans	1 cm-1 res	signal 3.71
p = 3 x 10-8 mbar	T = 140

17:45	C2H6_ASW_2021_09_23_0172

512 scans	1 cm-1 res	signal 3.71
p = 3 x 10-8 mbar	T = 140

Cryo off – Heater man 0

# Monday 27th Sep 2021	(VD) – C2H6:ASW 1:1 mixture at 20K

9:44 T = 278.1 K P = 1.4 * 10-8

9:45	Cryo-on

9:45	Laser On

10:43	MCT Detector cooled

11:35 T = 19.8K P = 1.7 * 10 e-9

11:40

## Background scan(s) #1

11:40	BG20210927_01

512 scans	res = 1 cm-1	signal = 3.66
               p = 1.7 * 10-9 mbar          T = 19.8

## Deposition : H2O:C2H6 @ 20K

12:35	20 min @ 1x10-7 mbar H2O

- Initial Temperature: 19.7K

- Initial pressure: 1.6 * 10-9 mbar

- Initial gas cell pressure: 1.011 Torr (0.560 ethane + 1.053 (-0.560) of water)

- Deposition pressure: 1 * 10-7

- Laser signal:  326.6 mV

- Deposition time: 20 min

- pressure after deposition 3.7 e-9

- final gas cell pressure = 0.968 Torr

12:56	 Head rotated Laser off

13:00	C2H6_ASW_2021_09_27_0001

512 scans	1 cm-1 res	signal 4.04
p = 2.4 x 10-9 mbar	T = 19.7K

13 :15 Macro 20 scans exit at loop 16

17:00	C2H6_ASW_2021_09_27_0018

512 scans	1 cm-1 res	signal 3.98
p = 1.3 x 10-9 mbar	T = 19.7K

17:15 Macro 20 scans exit at loop 10 up to scan

19:40 MCT Detector cooled

19:45	C2H6_ASW_2021_09_27_0029

512 scans	1 cm-1 res	signal 3.87
p = 1.3 x 10-9 mbar	T = 19.7K

20 :00 Macro 80 scans exit at loop 59 (up to scan 88)

28/09/2021

9:50 MCT Detector cooled

10:00	C2H6_ASW_2021_09_27_0089

512 scans	1 cm-1 res	signal 3.83
p = 1.1 x 10-9 mbar	T = 19.6K

10 :15 Macro 20 scans EXIT AT LOOP 17

14:20	C2H6_ASW_2021_09_27_0107

512 scans	1 cm-1 res	signal 3.83
p = 1.0 x 10-9 mbar	T = 19.6K

## Annealing to 25 K

14:35	C2H6_ASW_2021_09_27_0108

512 scans	1 cm-1 res	signal 3.81
p = 1.0 x 10-9 mbar	T = 19.6

14:50	C2H6_ASW_2021_09_27_0109

512 scans	1 cm-1 res	signal 3.81
p = 1.0 x 10-9 mbar	T = 25

15 :05 Macro 4 scans

## Annealing to 30 K

16:05	C2H6_ASW_2021_09_27_0114

512 scans	1 cm-1 res	signal 3.82
p = 1.1 x 10-9 mbar	T = 25

16:20	C2H6_ASW_2021_09_27_0115

512 scans	1 cm-1 res	signal 3.82
p = 1.1 x 10-9 mbar	T = 30

16:35	C2H6_ASW_2021_09_27_0116

512 scans	1 cm-1 res	signal 3.82
p = 1.1 x 10-9 mbar	T = 30

16 :50 Macro 2 scans

## Annealing to 40 K

17:20	C2H6_ASW_2021_09_27_0117

512 scans	1 cm-1 res	signal 3.83
p = 1.1 x 10-9 mbar	T = 30

17 :35 Macro 4 scans

18 :31 Macro 4 scans

19 :34 MCT Detector cooled

19 :40 Macro 80 scans exit at loop 62

29/09/2021

10:22 MCT Detector cooled

11:15	C2H6_ASW_2021_09_27_0190

512 scans	1 cm-1 res	signal 3.79
p = 1.1 x 10-9 mbar	T = 40k

11 :30 Macro 20 scans

16:45	C2H6_ASW_2021_09_27_0211

512 scans	1 cm-1 res	signal 3.82
p = 1.0 x 10-9 mbar	T = 40k

## Annealing to 60 K

17:05	C2H6_ASW_2021_09_27_0212

512 scans	1 cm-1 res	signal 3.83
p = 1.0 x 10-9 mbar	T = 40

17:20	C2H6_ASW_2021_09_27_0213

512 scans	1 cm-1 res	signal 3.83
p = 3.5 x 10-8 mbar	T = 59.7

17 : 36 MCT Detector refilled

17:45	C2H6_ASW_2021_09_27_0214

512 scans	1 cm-1 res	signal 3.83
p = 3.7 x 10-8 mbar	T = 60

Macro 80 scans exit at loop 70 up to scan 283

30/09/2021

10:22 MCT Detector cooled

10:30	C2H6_ASW_2021_09_27_0284 (unsaved for some reason)

512 scans	1 cm-1 res	signal 4.01
p = 2 x 10-8 mbar	T = 60

10:45	C2H6_ASW_2021_09_27_0284

512 scans	1 cm-1 res	signal 3.83
p = 1.3 x 10-8 mbar	T = 59.3

(T dropping down slightly)

## Annealing to 70 K

11:00	C2H6_ASW_2021_09_27_0285

512 scans	1 cm-1 res	signal 3.91
p = 2 x 10-8 mbar	T = 60

Macro 2 scans

## Annealing to 90 K

11:45	C2H6_ASW_2021_09_27_0288

512 scans	1 cm-1 res	signal 3.87
p = 1 x 10-7 mbar	T = 70

12 :00 Macro 4 scans

## Annealing to 120 K

13:05	C2H6_ASW_2021_09_27_0293

512 scans	1 cm-1 res	signal 3.81
p = 2.2 x 10-9 mbar	T = 90

13 :20 Macro 4 scans

## Annealing to 135 K

14:20	C2H6_ASW_2021_09_27_0298

512 scans	1 cm-1 res	signal 3.81
p = 2.8 x 10-9 mbar	T = 120

14 :35 Macro 20 scans exit at loop 7

16:15 MCT DETECTOR COOLED

16:20	C2H6_ASW_2021_09_27_0306

512 scans	1 cm-1 res	signal 3.76
p = 2.2 x 10-8 mbar	T = 135 k

Macro 70 scans (exit loop 61) up to scan 362 – 7:45

01/10/2021

10:37 MCT Detector colled

11:00	C2H6_ASW_2021_09_27_0363

512 scans	1 cm-1 res	signal 3.82
p = 6.7 x 10-9 mbar	T = 135 k

Collection stopped

Has the scan be saved ?

11:40	C2H6_ASW_2021_09_27_0363

512 scans	1 cm-1 res	signal 3.82
p = 6.7 x 10-9 mbar	T = 135 k

12:00	C2H6_ASW_2021_09_27_0364

512 scans	1 cm-1 res	signal 3.82
p = 6.7 x 10-9 mbar	T = 135 k

(lots of bad scans)

13:25	C2H6_ASW_2021_09_27_0365

512 scans	1 cm-1 res	signal 3.82
p = 6.1 x 10-9 mbar	T = 135 k

13:40	C2H6_ASW_2021_09_27_0366

512 scans	1 cm-1 res	signal 3.82
p = 6.0 x 10-9 mbar	T = 135 k

14 :00	Heater off cryo off

# Monday 04th Oct 2021	(VD) – C2H6 on top Ic 130K

9 :22 T = 278 P = 8.1 e-9 mbar

9:23 Cryo ON Laser On

11: 00 T = 19.8 P = 1.2 * 10-9 mbar

11:48 MCT Detector cooled

12:00 Heater set-up to 120K

12:05	Gas cell opened (0.947 initial P)

14:00

(decided to deposit at 130K)

Cf difference between 120K dep (10-14-blue) and 13K dep (10-30-red)

## Background scan(s) #1

14:30	BG20211004_01

512 scans	res = 1 cm-1	signal = 3.85
               p = 2.2 * 10-9 mbar          T = 130

15 :40 Head rotated (T = 130K P = 1.7 e-9)

## Deposition 1: H2O @ 130K

15:45 	20 min @ 1x10-7 mbar H2O

- Initial Temperature: 130K

- Initial pressure: 1.7 * 10-9 mbar

- Initial gas cell pressure: 7.47 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  323.8 mV

- Deposition time: 20 min

- pressure after deposition .13 e-8

- final gas cell pressure = 6.66 Torr

16:05 Head rotated

16:08	C2H6_ASW_2021_10_04_0001

512 scans	1 cm-1 res	signal 3.87
p = 8.0 x 10-9 mbar	T = 130K

(data aquisition stopped)

17:26	C2H6_ASW_2021_10_04_0001

512 scans	1 cm-1 res	signal 3.70
p = 4.0 x 10-9 mbar	T = 130K

18:04	C2H6_ASW_2021_10_04_0002

512 scans	1 cm-1 res	signal 3.88
p = 3.9 x 10-9 mbar	T = 130K

Cooling to 20K

18 :40 Heater on man 0

05/10/2021

11 :44 MCT Detector cooled

12:06	C2H6_ASW_2021_10_04_0003

512 scans	1 cm-1 res	signal 3.83
p = 7.5 x 10-10 mbar	T = 19.4K

12:20	C2H6_ASW_2021_10_04_0004

512 scans	1 cm-1 res	signal 3.83
p = 7.5 x 10-10 mbar	T = 19.4K

14:00	C2H6_ASW_2021_10_04_0005

512 scans	1 cm-1 res	signal 3.83
p = 7.5 x 10-10 mbar	T = 19.4K

14:15	C2H6_ASW_2021_10_04_0006

512 scans	1 cm-1 res	signal 3.83
p = 7.5 x 10-10 mbar	T = 19.4K

Ethane line filling procedure

- Gas cell pumping 0.56 Torr

- Close gas cell / Open ethane line for pumping (P in gas line increased to 2.1 e-1)

- Open regulator and then close regulator + ethane line

- Open ethane bottle and close

- Open regulator to introduce 1.4 bar

- Open gas cell chamber

- Close pump valve

- Open ethane black valve and fill gas line with ethane using leak valve

- Open pump valve and for time to pump

- Close pump valve and fill gas cell w ethane

14 :53	Head rotated

## Deposition 2: C2H6 @ 20K

16:30 	20 min @ 1x10-7 mbar H2O

- Initial Temperature: 19.3K

- Initial pressure: 7.2 * 10-10 mbar

- Initial gas cell pressure: 8.70 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  326.1 mV

- Deposition time: 20 min

- pressure after deposition 1.0 e-9

- final gas cell pressure = 8.45 Torr

16:58	Head rotated/Laser off

17:00	C2H6_ASW_2021_10_04_0007

512 scans	1 cm-1 res	signal 3.83
p = 7.1 x 10-10 mbar	T = 19.4K

17:15	C2H6_ASW_2021_10_04_0008

512 scans	1 cm-1 res	signal 3.92
p = 7 x 10-10 mbar	T = 19.4K

## Annealing to 30 K

17:30	C2H6_ASW_2021_10_04_0009

512 scans	1 cm-1 res	signal 3.95
p = 7 x 10-10 mbar	T = 19.4

Macro 3 scans

18:30	C2H6_ASW_2021_10_04_0013

512 scans	1 cm-1 res	signal 3.95
p = 7 x 10-10 mbar	T = 30

18:45	C2H6_ASW_2021_10_04_0014

512 scans	1 cm-1 res	signal 3.95
p = 7 x 10-10 mbar	T = 30

## Annealing to 40 K

19:00	C2H6_ASW_2021_10_04_0015

512 scans	1 cm-1 res	signal 3.95
p = 7 x 10-10 mbar	T = 30

19:15	C2H6_ASW_2021_10_04_0016

512 scans	1 cm-1 res	signal 3.95
p = 1.1 x 10-9 mbar	T = 39.8

19:30	C2H6_ASW_2021_10_04_0017

512 scans	1 cm-1 res	signal 3.95
p = 9.1 x 10-10 mbar	T = 40.0

## Annealing to 50 K

19:45	C2H6_ASW_2021_10_04_0018

512 scans	1 cm-1 res	signal 3.95
p = 7 x 10-10 mbar	T = 40

20:00	C2H6_ASW_2021_10_04_0019

512 scans	1 cm-1 res	signal 3.93
p = 1.3 x 10-9 mbar	T = 50

20 :16 MCT Detector refilled

20:20	C2H6_ASW_2021_10_04_0020

512 scans	1 cm-1 res	signal 3.87
p = 9.1 x 10-10 mbar	T = 50

## Annealing to 60 K

20:35	C2H6_ASW_2021_10_04_0021

512 scans	1 cm-1 res	signal 3.95
p = 7 x 10-10 mbar	T = 40

20 :50 Macro 80 scans exit at loop 59 up to scan 80

10:37 MCT Detector cooled

10:45	C2H6_ASW_2021_10_04_0081

512 scans	1 cm-1 res	signal 3.76
p = 2.6 x 10-8 mbar	T = 60

11:00	C2H6_ASW_2021_10_04_0082

512 scans	1 cm-1 res	signal 3.76
p = 2.6 x 10-8 mbar	T = 60

## Annealing to 65 K

11:15	C2H6_ASW_2021_10_04_0083

512 scans	1 cm-1 res	signal 3.75
p = 2.5 x 10-8 mbar	T = 60

11:30	C2H6_ASW_2021_10_04_0084

512 scans	1 cm-1 res	signal 3.75
p = 8.4 x 10-8 mbar	T = 65K

11 :45	Macro 8 scans

13:40	C2H6_ASW_2021_10_04_0093

512 scans	1 cm-1 res	signal 3.75
p = 2.2 x 10-8 mbar	T = 65K

## Annealing to 90 K

14:00	C2H6_ASW_2021_10_04_0094

512 scans	1 cm-1 res	signal 3.71
p = 1.2 x 10-8 mbar	T = 65K

14 :15 Macro 3 scans

## Annealing to 120 K

15:00	C2H6_ASW_2021_10_04_0098

512 scans	1 cm-1 res	signal 3.71
p = 1.9 x 10-9 mbar	T = 65K

15 :15 Macro 3 scans

## Annealing to 135 K

16:00	C2H6_ASW_2021_10_04_0102

512 scans	1 cm-1 res	signal 3.71
p = 1.9 x 10-9 mbar	T = 120K

16 :15 Macro 3 scans

## Annealing to 140 K

17:00	C2H6_ASW_2021_10_04_0106

512 scans	1 cm-1 res	signal 3.71
p = 1.1 x 10-8 mbar	T = 135K

17 :15	Macro 20 scans

## Annealing to 150 K

22:00	C2H6_ASW_2021_10_04_0127

512 scans	1 cm-1 res	signal 3.71
p = 2.7 x 10-8 mbar	T = 135K

22 :17 MCT Detector cooled

22 :20 Macro 70 scans exit at loop 52

07/10/2021

## Annealing to 160 K

10:30	C2H6_ASW_2021_10_04_0180

512 scans	1 cm-1 res	signal 3.67
p = 2.9 x 10-9 mbar	T = 150K

10 :45	Macro 2 scan

11 :15	Cryo off – Heater Man 0

11 :46 T set up 300K

12 :11 T = 298	P = 2.13 e-7

T set up Man 0

12:23	t = 288.9	P = 2.4 E-6

12:36 	T = 280.9	P = 1.2 e-7

# Thursday 07th Oct 2021 (VD) – C2H6 – ASW – C2H6 (Sandwish)

12:37 Cryo ON

13:56 	MCT Detector cooled

14:04	T = 19.8K	P = 3.1 e-9

15:26 T = 19.6 P = 2.2 * 10-9 mbar

15:26 Laser On

15:33 gas cell valve open (0.851)

15:51

## Background scan(s) #1

16:00	BG20211007_01

512 scans	res = 1 cm-1	signal = 3.62
               p = 2.0 * 10-9 mbar          T = 19.6

16 :21 Head rotated

Ethane line filling procedure

- Gas cell pumping 0.56 Torr

- Close gas cell / Open ethane line for pumping (P in gas line increased to 2.1 e-1)

- Open regulator and then close regulator + ethane line

- Open ethane bottle and close

- Open regulator to introduce 1.4 bar

- Open gas cell chamber

- Close pump valve

- Open ethane black valve and fill gas line with ethane using leak valve

- Open pump valve and for time to pump

- Close pump valve and fill gas cell w ethane

## Deposition 1: C2H6 @ 20K

16:40 	10 min @ 1x10-7 mbar H2O

- Initial Temperature: 19.6K

- Initial pressure: 1.8 * 10-9 mbar

- Initial gas cell pressure: 8.30 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  323.8 mV

- Deposition time: 10 min

- pressure after deposition 2.0 e-9

- final gas cell pressure = 8.17 Torr

16:51 Head rotated

16:55	C2H6_ASW_2021_10_07_0001

512 scans	1 cm-1 res	signal 3.70
p = 1.7 x 10-9 mbar	T = 19.6K

17 :10 Head rotated

## Deposition 2: H2O @ 20K

17:15 	20 min @ 1x10-7 mbar H2O

- Initial Temperature: 19.6K

- Initial pressure: 1.6 * 10-9 mbar

- Initial gas cell pressure: 8.38 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  325.9 mV

- Deposition time: 20 min

- pressure after deposition 1.3 e-8

- final gas cell pressure = 7.30 Torr

17:34 Head rotated / Gas cell pumped

17:40	C2H6_ASW_2021_10_07_0002

512 scans	1 cm-1 res	signal 3.73
p = 5.5 x 10-9 mbar	T = 19.7K

17 :56 Head rotated

## Deposition 3: C2H6 @ 20K

18:05 	10 min @ 1x10-7 mbar H2O

- Initial Temperature: 19.7K

- Initial pressure: 2.5 * 10-9 mbar

- Initial gas cell pressure: 9.14 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  323.6 mV

- Deposition time: 10 min

- pressure after deposition 2.0 e-9

- final gas cell pressure = 9.01 Torr

18:15 Head rotated

18:20	C2H6_ASW_2021_10_07_0003

512 scans	1 cm-1 res	signal 3.73
p = 1.9 x 10-9 mbar	T = 19.7K

18:35	C2H6_ASW_2021_10_07_0004

512 scans	1 cm-1 res	signal 3.68
p = 1.7 x 10-9 mbar	T = 19.7K

19:10	C2H6_ASW_2021_10_07_0005

512 scans	1 cm-1 res	signal 3.67
p = 1.5 x 10-9 mbar	T = 19.7K

08/10/2021

09 :52 Laser Off

09 :53 MCT Detector cooled

10:15	C2H6_ASW_2021_10_07_0006

512 scans	1 cm-1 res	signal 3.81
p = 6.1 x 10-10 mbar	T = 19.5K

Macro 2 scans

11:00	C2H6_ASW_2021_10_07_0009

512 scans	1 cm-1 res	signal 3.90
p = 6.2 x 10-10 mbar	T = 19.5K

## Annealing to 25 K

11:15	C2H6_ASW_2021_10_07_0010

512 scans	1 cm-1 res	signal 3.91
p = 6.1 x 10-10 mbar	T = 19.5K

Macro 3 scans

12:15	C2H6_ASW_2021_10_07_0014

512 scans	1 cm-1 res	signal 3.88
p = 1.3 x 10-9 mbar	T = 25K

## Annealing to 30 K

12:30	C2H6_ASW_2021_10_07_0015

512 scans	1 cm-1 res	signal 3.87
p = 1.1 x 10-9 mbar	T = 25K

12:45	C2H6_ASW_2021_10_07_0016

512 scans	1 cm-1 res	signal 3.87
p = 1.1 x 10-9 mbar	T = 30K

13 :15	Macro 4 scans

14:15	C2H6_ASW_2021_10_07_0021

512 scans	1 cm-1 res	signal 3.83
p = 7.5 x 10-10 mbar	T = 30K

14:30	C2H6_ASW_2021_10_07_0022

512 scans	1 cm-1 res	signal 3.83
p = 7.5 x 10-10 mbar	T = 30K

## Annealing to 40 K

14:45	C2H6_ASW_2021_10_07_0023

512 scans	1 cm-1 res	signal 3.84
p = 7.5 x 10-10 mbar	T = 30K

Macro 4 scans

## Annealing to 50 K

16:00	C2H6_ASW_2021_10_07_0028

512 scans	1 cm-1 res	signal 3.84
p = 8.0 x 10-10 mbar	T = 40K

16 :15	Macro 3 scan

17:00	C2H6_ASW_2021_10_07_0032

512 scans	1 cm-1 res	signal 3.84
p = 1 x 10-9 mbar	T = 50K

## Annealing to 60 K

17:15	C2H6_ASW_2021_10_07_0033

512 scans	1 cm-1 res	signal 3.84
p = 9.7 x 10-10 mbar	T = 50K

17:30	C2H6_ASW_2021_10_07_0034

512 scans	1 cm-1 res	signal 3.84
p = 3.9 x 10-8 mbar	T = 59.8K

17:45	C2H6_ASW_2021_10_07_0035

512 scans	1 cm-1 res	signal 3.84
p = 4.1 x 10-8 mbar	T = 60K

18 :02 MCT Detector cooled

18:10	C2H6_ASW_2021_10_07_0036

512 scans	1 cm-1 res	signal 3.75
p = 3.9 x 10-8 mbar	T = 60K

18 :24 Macro 60 scans end 08 :08 AM (09/10/2021) – up to scan 96

09/10/2021

13 :03 T = 60 p = 2.15 * 10-8 mbar

13 :20 MCT Detector cooled

13:30	C2H6_ASW_2021_10_07_0097

512 scans	1 cm-1 res	signal 3.76
p = 2.1 x 10-8 mbar	T = 60K

13:45	C2H6_ASW_2021_10_07_0098

512 scans	1 cm-1 res	signal 3.84
p = 2.1 x 10-8 mbar	T = 60K

## Annealing to 65 K

14:00	C2H6_ASW_2021_10_07_0099

512 scans	1 cm-1 res	signal 3.93
p = 2.1 x 10-8 mbar	T = 60K

14:15	C2H6_ASW_2021_10_07_0100

512 scans	1 cm-1 res	signal 3.93
p = 5.3 x 10-7 mbar	T = 65K

14 :30 Macro 4 scans

15:30	C2H6_ASW_2021_10_07_0105

512 scans	1 cm-1 res	signal 3.92
p = 7.3 x 10-8 mbar	T = 65K

## Annealing to 90 K

15:45	C2H6_ASW_2021_10_07_0106

512 scans	1 cm-1 res	signal 3.91
p = 6.1 x 10-8 mbar	T = 65K

16 :00	Macro 3 scans

16:45	C2H6_ASW_2021_10_07_0110

512 scans	1 cm-1 res	signal 3.88
p = 2.9 x 10-9 mbar	T = 90K

## Annealing to 120 K

17:00	C2H6_ASW_2021_10_07_0111

512 scans	1 cm-1 res	signal 3.88
p = 2.3 x 10-9 mbar	T = 90K

17 :15 Macro 2 scans

17:45	C2H6_ASW_2021_10_07_0114

512 scans	1 cm-1 res	signal 3.88
p = 4.6 x 10-9 mbar	T = 120K

## Annealing to 135 K

18:00	C2H6_ASW_2021_10_07_0115

512 scans	1 cm-1 res	signal 3.87
p = 3.6 x 10-9 mbar	T = 120K

18:15	C2H6_ASW_2021_10_07_0116

512 scans	1 cm-1 res	signal 3.87
p = 6 x 10-8 mbar	T = 134.8K

18 :31 MCT Detector cooled

18:40	C2H6_ASW_2021_10_07_0117

512 scans	1 cm-1 res	signal 3.82
p = 1.1 x 10-7 mbar	T = 135.2K

18 :55 Macro 65 scans (up to scan 182)

10/10/2021

11:36	MCT Detector cooled

Not good signal plus scan light on spectro blinking (quicker than usual)

11: 48	COMPUTER TURNED ON AND OFF

Problem solved

12:00	C2H6_ASW_2021_10_07_0183

512 scans	1 cm-1 res	signal 3.76
p = 1.0 x 10-8 mbar	T = 135.0K

12 :15	Macro 10 scans

14:45	C2H6_ASW_2021_10_07_0194

512 scans	1 cm-1 res	signal 3.80
p = 9.4 x 10-9 mbar	T = 135.0K

## Annealing to 140 K

15:00	C2H6_ASW_2021_10_07_0195

512 scans	1 cm-1 res	signal 3.80
p = 9.4 x 10-9 mbar	T = 135K

15 :15 Macro 3 scans

## Annealing to 150 K

16:00	C2H6_ASW_2021_10_07_0199

512 scans	1 cm-1 res	signal 3.77
p = 3.4 x 10-8 mbar	T = 140K

16 :15 Macro 10 scans exit at loop 9 up to scan

18:20	C2H6_ASW_2021_10_07_0209

512 scans	1 cm-1 res	signal 3.71
p = 4.2 x 10-9 mbar	T = 150K

18 :40 cryo off heater man 0

# Monday 11th Oct 2021 (VD) – ASW:C2H6 10:1 Mixture

10:18	T = 277.3K	P = 1.2 e-8

10:45 Cryo ON

10:45 Laser On

11:46 	MCT Detector cooled

11:47 T = 94.8 P = 2.1 e-9 mbar

13:11	T = 19.5 K	P = 1.3 * 10-9 mbar

## Background scan(s) #1

13:15	BG20211011_01

512 scans	res = 1 cm-1	signal = 3.71
               p = 1.3 * 10-9 mbar          T = 19.5

## Deposition : H2O:C2H6 (10:1) @ 20K

15:35	20 min @ 1x10-7 mbar H2O

- Initial Temperature: 19.5K

- Initial pressure: 1.1 * 10-9 mbar

- Initial gas cell pressure: 1.016 Torr

Procedure:

- Gas cell pumping 0.55 Torr

- Close gas cell / Open ethane line for pumping (P in gas line increased to 2.1 e-1)

- Open regulator and then close regulator + ethane line

- Open ethane bottle and close

- Open regulator to introduce 1.4 bar

- Open gas cell chamber

- Close pump valve

- Open ethane black valve and fill gas line with ethane using leak valve

- Open pump valve and for time to pump

- Close pump valve and fill gas cell w ethane 0.055 + 0.501 = 0.556

- Open water valve 0.556 + 0.499 = 1.055

- Pump valve open until we reached 0.152 Torr

- Water valve open up to 1.052

- Gas cell chamber valve closed

- Deposition pressure: 1 * 10-7

- Laser signal:  328.2 mV

- Deposition time: 20 min

- pressure after deposition 7.3 e-9

- final gas cell pressure = 0.920 Torr

15:55 Head rotated Laser off

16:00	C2H6_ASW_2021_10_11_0001

512 scans	1 cm-1 res	signal 3.38
p = 3.5 x 10-9 mbar	T = 19.6K

16:15	C2H6_ASW_2021_10_11_0002

512 scans	1 cm-1 res	signal 3.52
p = 1.8 x 10-9 mbar	T = 19.5K

16:30	C2H6_ASW_2021_10_11_0003

512 scans	1 cm-1 res	signal 3.52
p = 1.4 x 10-9 mbar	T = 19.5K

17:00	C2H6_ASW_2021_10_11_0004

512 scans	1 cm-1 res	signal 3.52
p = 1.1 x 10-9 mbar	T = 19.5K

## Annealing to 30 K

17:15	C2H6_ASW_2021_10_11_0005

512 scans	1 cm-1 res	signal 3.52
p = 1.1 x 10-9 mbar	T = 19.5K

17:30	C2H6_ASW_2021_10_11_0006

512 scans	1 cm-1 res	signal 3.52
p = 1.4 x 10-9 mbar	T = 29.9K

17:45	C2H6_ASW_2021_10_11_0007

512 scans	1 cm-1 res	signal 3.52
p = 1.2 x 10-9 mbar	T = 29.9K

## Annealing to 40 K

18:00	C2H6_ASW_2021_10_11_0008

512 scans	1 cm-1 res	signal 3.50
p = 1.2 x 10-9 mbar	T = 30K

18 :15 Macro 20 scans exit at loop 17 up to scan 25

22:16 MCT Detector cooled

22:20	C2H6_ASW_2021_10_11_0026

512 scans	1 cm-1 res	signal 3.50
p = 9.2 x 10-10 mbar	T = 40K

22 :40 Macro 65 scans exit at loop 57 up to scan 83

12/10/2021

12 :00 MCT Detector cooled

12:15	C2H6_ASW_2021_10_11_0084

512 scans	1 cm-1 res	signal 3.51
p = 6.7 x 10-10 mbar	T = 40K

12 :30 Macro 10 scans exit at loop 6

## Annealing to 50 K

14:00	C2H6_ASW_2021_10_11_0091

512 scans	1 cm-1 res	signal 3.49
p = 6.8 x 10-10 mbar	T = 40K

Macro 15 scans exit at loop 13

## Annealing to 60 K

17:20	C2H6_ASW_2021_10_11_0105

512 scans	1 cm-1 res	signal 3.49
p = 7.6 x 10-10 mbar	T = 50K

17:35	C2H6_ASW_2021_10_11_0106

512 scans	1 cm-1 res	signal 3.49
p = 1 x 10-9 mbar	T = 50K

17 :54	T = 60	P = 8 e-10

17 :55 Macro 65 scans up to scan 171

13/10/2021

10 :46 MCT Detector cooled (not full – not enough lN2)

10:50	C2H6_ASW_2021_10_11_0172

512 scans	1 cm-1 res	signal 3.30
p = 6 x 10-10 mbar	T = 60K

11:15	C2H6_ASW_2021_10_11_0173

512 scans	1 cm-1 res	signal 3.56
p = 6.2 x 10-10 mbar	T = 60K

11:30	C2H6_ASW_2021_10_11_0174

512 scans	1 cm-1 res	signal 3.56
p = 6.0 x 10-10 mbar	T = 60K

11:50	C2H6_ASW_2021_10_11_0175

512 scans	1 cm-1 res	signal 3.49
p = 6.0 x 10-10 mbar	T = 60K

## Annealing to 65 K

11:50	C2H6_ASW_2021_10_11_0176

512 scans	1 cm-1 res	signal 3.57
p = 6.0 x 10-10 mbar	T = 60K

12 :20 Macro 10 scans

## Annealing to 90 K

11:50	C2H6_ASW_2021_10_11_0187

512 scans	1 cm-1 res	signal 3.53
p = 7.5 x 10-10 mbar	T = 65K

15 :00 Macro 4 scans

16:00	C2H6_ASW_2021_10_11_0192

512 scans	1 cm-1 res	signal 3.53
p = 1.4 x 10-9 mbar	T = 90K

## Annealing to 120 K

16:20	C2H6_ASW_2021_10_11_0193

512 scans	1 cm-1 res	signal 3.52
p = 1.3 x 10-9 mbar	T = 90K

16 :35 Macro 3 scans

17:20	C2H6_ASW_2021_10_11_0197

512 scans	1 cm-1 res	signal 3.53
p = 3.4 x 10-9 mbar	T = 120K

## Annealing to 135 K

17:35	C2H6_ASW_2021_10_11_0198

512 scans	1 cm-1 res	signal 3.52
p = 1.3 x 10-9 mbar	T = 120K

17:50	C2H6_ASW_2021_10_11_0199

512 scans	1 cm-1 res	signal 3.52
p = 8 x 10-8 mbar	T = 134.8K

18 :05 MCT Detector cooled

18:15	C2H6_ASW_2021_10_11_0200

512 scans	1 cm-1 res	signal 3.52
p = 9 x 10-8 mbar	T = 135K

18 :30 Macro 65 scan up to 10 :11 scan 265

14 /10/2021

10 :49 MCT Detector cooled

11:00	C2H6_ASW_2021_10_11_0266

512 scans	1 cm-1 res	signal 3.65
p = 9.9 x 10-9 mbar	T = 135K

11 :15 Macro 20 scans

16:00	C2H6_ASW_2021_10_11_0287

512 scans	1 cm-1 res	signal 3.63
p = 9.9 x 10-9 mbar	T = 135K

16:15	C2H6_ASW_2021_10_11_0288

512 scans	1 cm-1 res	signal 3.63
p = 9.9 x 10-9 mbar	T = 135K

16:30	C2H6_ASW_2021_10_11_0289

512 scans	1 cm-1 res	signal 3.62
p = 9.8 x 10-9 mbar	T = 135K

*

17:00	C2H6_ASW_2021_10_11_0290

512 scans	1 cm-1 res	signal 3.62
p = 9.7 x 10-9 mbar	T = 135K

(lots of timeout !! )

17 :25 Macro 20 scans

15/10/2021

10 :50	MCT Detector cooled

11:00	C2H6_ASW_2021_10_11_0311

512 scans	1 cm-1 res	signal 3.38
p = 7.2 x 10-9 mbar	T = 135K

11:15	C2H6_ASW_2021_10_11_0312

512 scans	1 cm-1 res	signal 3.38
p = 7.2 x 10-9 mbar	T = 135K

11:30	C2H6_ASW_2021_10_11_0313

512 scans	1 cm-1 res	signal 3.66
p = 7.2 x 10-9 mbar	T = 135K

11:45	C2H6_ASW_2021_10_11_0313

512 scans	1 cm-1 res	signal 3.38
p = 7.2 x 10-9 mbar	T = 135K

12 :00 Macro 4 scans

13 :00 Macro 4 scans

## Annealing to 140 K

14:15	C2H6_ASW_2021_10_11_0323

512 scans	1 cm-1 res	signal 3.52
p = 7.1 x 10-9 mbar	T = 135K

14 :30 Macro 4 scans

15:30	C2H6_ASW_2021_10_11_0328

512 scans	1 cm-1 res	signal 3.59
p = 2.8 x 10-8 mbar	T = 140K

## Annealing to 150 K

15:45	C2H6_ASW_2021_10_11_0329

512 scans	1 cm-1 res	signal 3.59
p = 7.1 x 10-9 mbar	T = 135K

16 :00 Macro 20 scans exit at loop 15 up to scan 344

19:40 cryo off heater off

# Monday 18th Oct 2021 (VD) –

10:18	T = 278K	P = 5.6 e-9

10:22 Cryo ON

10:22 Laser On

11:10 	MCT Detector cooled

11:13 T = 137.3 P = 1.5 e-9 mbar

12:02 T = 19.7	P = 8.7 * 10-10

14:59 T = 19.5 	P= 8 e-10 mbar

15:00 T set up to 100K gas cell pumping

No background taken CO2 level too high …

19/10/2021

10:55	T = 277K	P = 5.1 e-9

10:56 Cryo ON

10:56 Laser On

11:30 	MCT Detector cooled

13:10 T = 19.5K	P = 7.4 * 10-10

T set-up 100K

## Background scan(s) #1

13:45	BG20211018_01

512 scans	res = 1 cm-1	signal = 3.69
               p = 2.3 * 10-9 mbar          T = 100K

14 :10 Head rotated

## Deposition 1: H2O @ 100K  np ASW

14:15 	20 min @ 1x10-7 mbar H2O

- Initial Temperature: 100K

- Initial pressure: 1.5 * 10-9 mbar

- Initial gas cell pressure: (0.55 initial) 7.25 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  327.8 mV

- Deposition time: 20 min

- pressure after deposition 8.3 e-9

- final gas cell pressure = 6.46 Torr

14:35 Head rotated / Gas cell pumped

14:45	C2H6_ASW_2021_10_18_0001

512 scans	1 cm-1 res	signal 3.77
p = 2.7 x 10-9 mbar	T = 100K

15 :02 Heater set to man 0

15 :21 	T = 19.5K	P = 9.3 * 10-10 mbar

15:30	C2H6_ASW_2021_10_18_0002

512 scans	1 cm-1 res	signal 3.75
p = 9.0 x 10-9 mbar	T = 19.4K

15:52	C2H6_ASW_2021_10_18_0003

512 scans	1 cm-1 res	signal 3.75
p = 8.0 x 10-9 mbar	T = 19.3K

16:10 Head rotated

## Deposition 2: C2H6 @ 20K

16:25 	10 min @ 1x10-7 mbar C2H6

- Initial Temperature: 19.2K

- Initial pressure: 8 * 10-10 mbar

- Initial gas cell pressure: (0.56 initial) 7.81 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  326.5 mV

- Deposition time: 10 min

- pressure after deposition 1.5 e-9

- final gas cell pressure = 7.51 Torr

16:36 Head rotated / Laser off

16:40	C2H6_ASW_2021_10_18_0004

512 scans	1 cm-1 res	signal 3.86
p = 1.0 x 10-9 mbar	T = 19.2K

17:00	C2H6_ASW_2021_10_18_0005

512 scans	1 cm-1 res	signal 3.86
p = 7.8 x 10-10 mbar	T = 19.1K

## Annealing to 30 K

17:15	C2H6_ASW_2021_10_18_0006

512 scans	1 cm-1 res	signal 3.87
p = 7.7 x 10-10 mbar	T = 19.1K

(pressure remain high (e-7) during whole scan )

17:30 Macro 4 scans

18:30	C2H6_ASW_2021_10_18_0011

512 scans	1 cm-1 res	signal 3.82
p = 7.4 x 10-8 mbar	T = 30K

## Annealing to 35 K

18:45	C2H6_ASW_2021_10_18_0012

512 scans	1 cm-1 res	signal 3.87
p = 5.2 x 10-8 mbar	T = 30K

19 :00 Macro 4 scans

20:00	C2H6_ASW_2021_10_18_0017

512 scans	1 cm-1 res	signal 3.87
p = 5.2 x 10-8 mbar	T = 35K

20 :17 MCT Detector refilled

20:20	C2H6_ASW_2021_10_18_0018

512 scans	1 cm-1 res	signal 3.75
p = 2.4 x 10-8 mbar	T = 35K

20 :35 Macro 65 scans exit at loop 60 up to scan 78

20/10

10:47	MCT Detector cooled

11:00	C2H6_ASW_2021_10_18_0079

512 scans	1 cm-1 res	signal 3.74
p = 1.2 x 10-9 mbar	T = 35K

## Annealing to 40 K

11:15	C2H6_ASW_2021_10_18_0080

512 scans	1 cm-1 res	signal 3.73
p = 5.2 x 10-8 mbar	T = 35K

11 :30 Macro 6 scans

13:00	C2H6_ASW_2021_10_18_0087

512 scans	1 cm-1 res	signal 3.73
p = 3.6 x 10-9 mbar	T = 40K

13 :15 Macro 6 scans

## Annealing to 50 K

14:45	C2H6_ASW_2021_10_18_0094

512 scans	1 cm-1 res	signal 3.73
p = 2 x 10-9 mbar	T = 45K

15 :00 Macro 6 scans

## Annealing to 60 K

16:30	C2H6_ASW_2021_10_18_0101

512 scans	1 cm-1 res	signal 3.70
p = 1.3 x 10-9 mbar	T = 50K

16:45	C2H6_ASW_2021_10_18_0102

512 scans	1 cm-1 res	signal 3.70
p = 3.8 x 10-8 mbar	T = 60K

17:00	C2H6_ASW_2021_10_18_0103

512 scans	1 cm-1 res	signal 3.70
p = 3.6 x 10-8 mbar	T = 60K

17:15	C2H6_ASW_2021_10_18_0104

512 scans	1 cm-1 res	signal 3.70
p = 3.5 x 10-8 mbar	T = 60K

## Annealing to 65 K

17:30	C2H6_ASW_2021_10_18_0105

512 scans	1 cm-1 res	signal 3.70
p = 3.3 x 10-9 mbar	T = 60K

17:45	C2H6_ASW_2021_10_18_0106

512 scans	1 cm-1 res	signal 3.68
p = 1.8 x 10-7 mbar	T = 65K

18:00	C2H6_ASW_2021_10_18_0107

512 scans	1 cm-1 res	signal 3.68
p = 2.7 x 10-8 mbar	T = 65K

## Annealing to 135 K

18:15	C2H6_ASW_2021_10_18_0108

512 scans	1 cm-1 res	signal 3.68
p = 2.4 x 10-8 mbar	T = 65K

18:30	C2H6_ASW_2021_10_18_0109

512 scans	1 cm-1 res	signal 3.68
p = 4 x 10-8 mbar	T = 135K

18 :46 MCT Detector cooled

18 :50 Macro 65 scans up to 9 :41 (scan 174)

10:52 MCT Detector cooled

11:00	C2H6_ASW_2021_10_18_0175

512 scans	1 cm-1 res	signal 3.71
p = 8.6 x 10-9 mbar	T = 135K

11:15	C2H6_ASW_2021_10_18_0176

512 scans	1 cm-1 res	signal 3.71
p = 8.4 x 10-9 mbar	T = 135K

11:30	C2H6_ASW_2021_10_18_0177

512 scans	1 cm-1 res	signal 3.71
p = 8.3 x 10-9 mbar	T = 135K

11 :45	Macro 4 scan

## Annealing to 140 K

12:45	C2H6_ASW_2021_10_18_0182

512 scans	1 cm-1 res	signal 3.68
p = 7.8 x 10-9 mbar	T = 135K

13 :00 Macro 4 scans

## Annealing to 150 K

14:00	C2H6_ASW_2021_10_18_0187

512 scans	1 cm-1 res	signal 3.65
p = 2.7 x 10-8 mbar	T = 140K

14 :15 Macro 20 scans up to scan 207

# Friday 22th Oct 2021 (VD) – ASW:C2H6 10:1 Mixture deposited at 100K

10:07	T = 274.7K	P = 7 e-9

10:08 Cryo ON

10:08 Laser On

11:05 	MCT Detector cooled

11:07	T = 99.6		P = 1.5 * 10-9

Procedure:

- Gas cell pumping 0.55 Torr

- Close gas cell / Open ethane line for pumping (P in gas line increased to 2.1 e-1)

- Make sure water needle valve pumped and close (before ethane intro)

- Open regulator and then close regulator + ethane line

- Open ethane bottle and close

- Open regulator to introduce 1.4 bar

- Open gas cell chamber

- Close pump valve

- Open ethane black valve and fill gas line with ethane using leak valve

- Open pump valve and for time to pump

- Close pump valve and fill gas cell w ethane 0.057 + 0.499 = 0.556

- Open water valve 0.556 + 0.500 = 1.056

- Pump valve open until we reached 0.157 Torr

- Water valve open up to 1.060

- Gas cell chamber valve closed

11:30	T = 19.5	 P = 9.2 * 10-10

T set-up to 100K

## Background scan(s) #1

12:00	BG20211022_01

512 scans	res = 1 cm-1	signal = 3.71
               p = 2.4 * 10-9 mbar          T = 100K

12 :15 Head rotated

## Deposition: H2O:C2H6 10:1 @ 100K

12:20 	20 min @ 1x10-7 mbar H2O

- Initial Temperature: 100K

- Initial pressure: 1.7 * 10-9 mbar

- Initial gas cell pressure: 1.002 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  328.3 mV

- Deposition time: 20 min

- pressure after deposition 3.8 e-9

- final gas cell pressure = 0.990 Torr

12:40 Head rotated / Gas cell pumped

12:40	C2H6_ASW_2021_10_22_0001

512 scans	1 cm-1 res	signal 3.69
p = 2.5 x 10-9 mbar	T = 100K

12 :55 Macro 6 scans

## Annealing to 135 K

14:20	C2H6_ASW_2021_10_22_0008

512 scans	1 cm-1 res	signal 3.66
p = 1.1 x 10-9 mbar	T = 100K

14 :35 Macro 30 scans exit at loop 12

## Annealing to 140 K

17:30	C2H6_ASW_2021_10_22_0021

512 scans	1 cm-1 res	signal 3.65
p = 1.5 x 10-8 mbar	T = 135K

17:45	C2H6_ASW_2021_10_22_0022

512 scans	1 cm-1 res	signal 3.65
p = 5.6 x 10-8 mbar	T = 140K

18:00	C2H6_ASW_2021_10_22_0023

512 scans	1 cm-1 res	signal 3.65
p = 4.8 x 10-8 mbar	T = 140K

18:15	C2H6_ASW_2021_10_22_0024

512 scans	1 cm-1 res	signal 3.65
p = 3.7 x 10-8 mbar	T = 140K

Heater off / cryo off

# Sunday 24th Oct 2021 (VD) –

11:22	T = 278.1K	P = 4.2 * 10 e-9

11:23 Cryo ON

11:23 Laser On

11:48 	MCT Detector cooled

12:20	T = 112	p = 1.1 * 10-9

12:50	T = 19.7		P = 6.7 * 10-10

## Background scan(s) #1

13:05	BG20211024_01

512 scans	res = 1 cm-1	signal = 3.69
               p = 6.3 * 10-10 mbar          T = 19.6K

13 :20 Head rotated

## Deposition 1: H2O @ 20K

13:30 	20 min @ 1x10-7 mbar H2O

- Initial Temperature: 19.6K

- Initial pressure: 6.3 * 10-9 mbar

- Initial gas cell pressure: (0.55 initial) 7.45 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  327.8 mV

- Deposition time: 20 min

- pressure after deposition 7 * e-9

- final gas cell pressure = 6.36 Torr

13:50 Head rotated / Gas cell pumped

13:55	C2H6_ASW_2021_10_24_0001

512 scans	1 cm-1 res	signal 3.69
p = 4 x 10-9 mbar	T = 19.6K

14:10	C2H6_ASW_2021_10_24_0002

512 scans	1 cm-1 res	signal 3.69
p = 1.4 x 10-9 mbar	T = 19.6K

## Annealing to 100 K

14:30	C2H6_ASW_2021_10_24_0003

512 scans	1 cm-1 res	signal 3.68
p = 1.0 x 10-10 mbar	T = 19.1K

14:45	C2H6_ASW_2021_10_24_0004

512 scans	1 cm-1 res	signal 3.68
p = 2.6 x 10-9 mbar	T = 99.4K

15:00	C2H6_ASW_2021_10_24_0005

512 scans	1 cm-1 res	signal 3.68
p = 1.8 x 10-9 mbar	T = 99.9K

## Cooling to 20 K

15:15 Heater man 0

15:15	C2H6_ASW_2021_10_24_0006

512 scans	1 cm-1 res	signal 3.66
p = 1.5 x 10-9 mbar	T = 100K

15:30	C2H6_ASW_2021_10_24_0007

512 scans	1 cm-1 res	signal 3.64
p = 9.2 x 10-10 mbar	T = 35K

15:45	C2H6_ASW_2021_10_24_0008

512 scans	1 cm-1 res	signal 3.64
p = 6.7 x 10-10 mbar	T = 19.5K

16 :00 Macro 50 scan

25/10/2021

10 :15 MCT Detector cooled

10:30	C2H6_ASW_2021_10_24_0059

512 scans	1 cm-1 res	signal 3.35
p = 5 x 10-10 mbar	T = 19.2K

10:45	C2H6_ASW_2021_10_24_0060

512 scans	1 cm-1 res	signal 3.63
p = 5 x 10-10 mbar	T = 19.2K

11:00	C2H6_ASW_2021_10_24_0061

512 scans	1 cm-1 res	signal 3.66
p = 5 x 10-10 mbar	T = 19.2K

11 :20 Head rotated

## Deposition 2: C2H6 @ 20K

11:30 	10 min @ 1x10-7 mbar C2H6

- Initial Temperature: 19.1K

- Initial pressure: 5 * 10-10 mbar

- Initial gas cell pressure: (0.55 initial) 8.07 Torr

- Deposition pressure: 1 * 10-7

- Laser signal:  324.7 mV

- Deposition time: 10 min

- pressure after deposition 1 e-9

- final gas cell pressure = 7.75 Torr

11:41 Head rotated / Laser off

11:45	C2H6_ASW_2021_10_24_0062

512 scans	1 cm-1 res	signal 3.74
p = 7 x 10-10 mbar	T = 19.1K

Macro 4 scans

## Annealing to 30 K

13:15	C2H6_ASW_2021_10_24_0067

512 scans	1 cm-1 res	signal 3.73
p = 5 x 10-10 mbar	T = 19.1K

Time outs …

13 :35 Macro 3 scans

14:35	C2H6_ASW_2021_10_24_0071

512 scans	1 cm-1 res	signal 3.71
p = 2.1 x 10-7 mbar	T = 30K

14:50	C2H6_ASW_2021_10_24_0072

512 scans	1 cm-1 res	signal 3.71
p = 1.7 x 10-7 mbar	T = 30K

## Annealing to 40 K

15:15	C2H6_ASW_2021_10_24_0073

512 scans	1 cm-1 res	signal 3.66
p = 1.1 x 10-7 mbar	T = 30K

15 :30	Macro 3 scans

16:20	C2H6_ASW_2021_10_24_0077

512 scans	1 cm-1 res	signal 3.66
p = 8.5 x 10-9 mbar	T = 40K

## Annealing to 50 K

16:40	C2H6_ASW_2021_10_24_0078

512 scans	1 cm-1 res	signal 3.66
p = 6.0 x 10-9 mbar	T = 40K

Macro 3 scans (probelem with number of scan !!)

18:00	C2H6_ASW_2021_10_24_0082

512 scans	1 cm-1 res	signal 3.66
p = 3.2 x 10-9 mbar	T = 50K

## Annealing to 60 K

18:15	C2H6_ASW_2021_10_24_0083

512 scans	1 cm-1 res	signal 3.65
p = 2.6 x 10-9 mbar	T = 50K

Macro 2 scans

19:00	C2H6_ASW_2021_10_24_0086

512 scans	1 cm-1 res	signal 3.65
p = 6 x 10-9 mbar	T = 60K

19:15	C2H6_ASW_2021_10_24_0087

512 scans	1 cm-1 res	signal 3.63
p = 5.8 x 10-9 mbar	T = 60K

Macro 10 scans

22:50	C2H6_ASW_2021_10_24_0098

512 scans	1 cm-1 res	signal 3.63
p = 5.8 x 10-9 mbar	T = 60K

## Annealing to 135 K

23:10	C2H6_ASW_2021_10_24_0099

512 scans	1 cm-1 res	signal 3.65
p = 3.7 x 10-9 mbar	T = 60K

23:23	C2H6_ASW_2021_10_24_0100

512 scans	1 cm-1 res	signal 3.65
p = 2.7 x 10-8 mbar	T = 132.9K

23 :44 MCT Detector cooled

23 :45	Macro 60 scan exit at loop 51

12:40 MCT Detector cooled

12:45	C2H6_ASW_2021_10_24_0152

512 scans	1 cm-1 res	signal 3.63
p = 9.4 x 10-9 mbar	T = 135K

Macro 20 scans

## Annealing to 140 K

17:00	C2H6_ASW_2021_10_24_0168

512 scans	1 cm-1 res	signal 3.65
p = 8.6 x 10-9 mbar	T = 135K

Macro 3 scans

## Annealing to 150 K

18:00	C2H6_ASW_2021_10_24_0172

512 scans	1 cm-1 res	signal 3.65
p = 8.6 x 10-9 mbar	T = 135K

Macro 10 scans

Time outs (rate not accurate)

22:52	C2H6_ASW_2021_10_24_0183

512 scans	1 cm-1 res	signal 3.65
p = 2.9 x 10-7 mbar	T = 150K

# Thursday 28th Oct 2021 (VD – AD - PAH) ASW:C2H6 mixture	5:1

12:37 Cryo ON yesterday

09:51 	MCT Detector cooled

09:52	T = 19.4K	P = 5  e-10 mbar

15:26 Laser On

## Background scan(s) #1

10:21	BG20211028_01

256 scans	res = 1 cm-1	signal = 4.67
               p = 5.0 * 10-10 mbar          T = 19.5

Think About click “Collect background before every sample”

You will need to save it manually

“Use the specified BG file”.

## Deposition: ASW:C2H6 5:1 @ 20K

16:40 	20 min @ 1x10-7 mbar H2O:C2H6 (5:1)

- Initial Temperature: 19.5K

- Initial pressure: 5 * 10-10 mbar

Gas cell filling:

Init P (zero offset): 0.055

Ethane intro: 0.555

Water intro: 1.035

Pumped down to: 0.235

Water intro 2: 1.035

Gas cell P (final): 1.000

- Deposition pressure: 1 * 10-7

- Laser signal:  328.2 mV

- Deposition time: 10 min

- final gas cell pressure = 0.880Torr

11:47 Head rotated?Laser turned off

11:50	C2H6_ASW_2021_10_28_0002

216 scans	1 cm-1 res	signal 4.16
p = 3.8 x 10-9 mbar	T = 19.5K

13:16	C2H6_ASW_2021_10_28_0003

216 scans	1 cm-1 res	signal 4.16
p = 6.1 x 10-10 mbar	T = 19.5K

## Electron IRRADIATION

Turn surface 90 degree in opposite direction than for deposition (down to 270)

Turn Electron Gun (EG) On

Press Resume (Saved setting)

ALLOW TIME FOR IT TO WARM UP (10 microA)

13:39 Pressure start rising (10 – 11 micro amps)

Irradiation spot (to check)

13:41	P= 1.3 * 10-7 mbar

13:55	P = 3 e-7	T = 19.6K

13:59 electron gun turned off

14:00 Head rotated

14:00	C2H6_ASW_2021_10_28_0004

216 scans	1 cm-1 res	signal 3.62
p = 1.1 x 10-7 mbar	T = 19.5K

## Warm up to 120K

14:20	C2H6_ASW_2021_10_28_0005

216 scans	1 cm-1 res	signal 3.62

14:40	C2H6_ASW_2021_10_28_0006

216 scans	1 cm-1 res	signal 3.62

15:45	C2H6_ASW_2021_10_28_0007

216 scans	1 cm-1 res	signal 3.62
p = 7.5 x 10-9 mbar	T = 120K

16:05	C2H6_ASW_2021_10_28_0008

216 scans	1 cm-1 res	signal 3.62
p = 6.1 x 10-9 mbar	T = 120K

## Warm up to 135K

16:40	C2H6_ASW_2021_10_28_0009

216 scans	1 cm-1 res	signal 3.62

17:00	C2H6_ASW_2021_10_28_0010

216 scans	1 cm-1 res	signal 3.62

17:20	C2H6_ASW_2021_10_28_0011

216 scans	1 cm-1 res	signal 3.65

17 :40 MCT Detector cooled

17:40	C2H6_ASW_2021_10_28_0012

216 scans	1 cm-1 res	signal 3.60

18 :00 Macro 50 scans

29/10/2021

10 :30 Computer was turned off (Macro started when comp turned on at loop 4)

10 :40 Macro exit (loop 4 signal 1.58) up to scan 16

11 :05 MCT Detector cooled

11:15	C2H6_ASW_2021_10_28_0017

216 scans	1 cm-1 res	signal 3.67
p = 8.5 x 10-9 mbar	T = 135K

11 :35 Macro 10 scans (again, Macro stop when computer go to sleep mode)

14 :45 Heater man 0 / cryo off